| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 17 to Document 41-E |
|  | 22 August 2007 |
|  | Original: English |
| Asia-Pacific Telecommunity Administrations |  |
| COMMON proposals for the work of the conference |  |
| Agenda item 1.17 |  |

1.17	to consider the results of ITU-R studies on compatibility between the fixed-satellite service and other services around 1.4 GHz, in accordance with Resolution 745 (WRC-03)

Introduction

Agenda item 1.17 addresses protection of existing services in all Regions from non-geostationary-satellite networks in the fixed-satellite service using the frequency bands around 1.4 GHz on a secondary basis, in accordance with Resolution 745 (WRC-03).

Under this agenda item, compatibility studies between the fixed-satellite service (FSS) for feeder links to non-geostationary mobile-satellite service (MSS) satellites and other services around 1.4 GHz had been conducted in ITU-R.

APT members note that, based on ITU-R studies, sharing between FSS feeder link and other services is not feasible, because large separation distance, geographical separation, or additional constrains are required.

Therefore, APT members support Method 1 of the CPM Report, that is, suppression of the secondary FSS allocation for MSS feeder links in the frequency bands 1 390-1 392 MHz (Earth-to-space) and 1 430-1 432 MHz (space-to-Earth).

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	ASP/41A17/316

1 300-1 525 MHz

| SUP ASP/41A17/317 Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 350-1 400 FIXED MOBILE RADIOLOCATION | 1 350-1 400 RADIOLOCATION |  |
| 5.149 5.338 5.339 | 5.149 5.334 5.339 |  |
| 1 400-1 427 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.341 |  |  |
| 1 427-1 429 SPACE OPERATION (Earth-to-space) FIXED MOBILE except aeronautical mobile 5.341 |  |  |
| 1 429-1 452 FIXED MOBILE except aeronautical mobile | 1 429-1 452 FIXED MOBILE 5.343 |  |
| 5.341 5.342 | 5.341 |  |

5.339A

Reasons:	No. 5.339A prescribes only additional allocation of the secondary FSS allocation for MSS feeder links in the frequency bands and sharing study of this agenda item resulted that sharing between FSS and other services is not feasible in this band. Therefore, if Method 1, which APT members support and is the only method which is proposed as a result of the study of this agenda item, is adopted, RR No. 5.339A is not necessary any more.

SUP	ASP/41A17/318

RESOLUTION  745  (WRC-03)

Protection of existing services in all Regions from non-geostationary-satellite
networks in the fixed-satellite service using the frequency bands 
around 1.4 GHz on a secondary basis

Reasons:	Consequential to the suppression of the secondary FSS allocation for MSS feeder links.

_____________