| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 17 to Document 39-E |
|  | 20 August 2007 |
|  | Original: Arabic |
| Arab States Common Proposals |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| Agenda item 1.18 |  |

1.18	to review pfd limits in the band 17.7-19.7 GHz for satellite systems using highly inclined orbits, in accordance with Resolution 141 (WRC-03)

Introduction

Pursuant to Resolution 141 (WRC-03), ITU-R has conducted studies to determine whether the current power flux density (pfd) limits in RR Article 21 for non-GSO systems in the fixed-satellite service (FSS) using highly inclined orbits (HIO) having an apogee altitude greater than 18 000 km and orbital inclination between 35° and 145° are adequate to protect the fixed service (FS) in the 17.7-19.7 GHz band.

1)	The Arab States propose to add in Article 21 of the Radio Regulations a more stringent pfd mask applicable to HIO satellites in the 17.7-19.7 GHz band, and consequently, to suppress Resolution 141 as the studies in question have already been completed.

MOD	ARB/39A17/1

2)	The Arab States prefer a mask whose lower limit is –125 dB(W/m2) for an angle of arrival (δ) of 0°-5° above the horizontal plane, and the consequential changes for angles 5°-25° and 25°-90°. However, the Arab States are ready to accept the limit of 
–123 dB(W/m2) indicated in the proposed Method C for an angle of 0°-5° and the consequential changes for other angles, subject to the inclusion of the following designation in RR Table 21-4, in conformity with the text of this agenda item:

Fixed-satellite (space-to-Earth) (highly inclined satellite orbit)

SUP	ARB/39A17/2

RESOLUTION  141  (WRC-03)

Sharing between certain types of non-geostationary-satellite systems
in the fixed-satellite service and stations in the fixed service
in the 17.7-19.7 GHz band

Reasons:	To satisfy the requirements of this agenda item.

______________