| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 4 to Document 39-E |
|  | 20 August 2007 |
|  | Original: Arabic |
| Arab States Common Proposals |  |
| proposals for the work of the conference |  |
| Agenda item 1.5 |  |

1.5	to consider spectrum requirements and possible additional spectrum allocations for aeronautical telecommand and high bit-rate aeronautical telemetry, in accordance with Resolution 230 (WRC-03)

Introduction

With the increasing complexity of aircraft design and pressure to shorten time-scales for the development of new aircraft, there is an increasing demand for access to spectrum for the provision of aeronautical telemetry and telecommand systems. WRC-07 Agenda item 1.5 seeks to identify spectrum that can be used to meet this demand.

Proposals

The Arab States propose the addition in RR Article 5 of the aeronautical mobile service (AMS) in the band 5 091-5 150 MHz limited to aeronautical mobile telemetry for flight testing (air-to-ground) and always giving priority to the microwave landing system (MLS), subject to the condition that flight testing does not claim protection from nor cause harmful interference to the MLS in the future within this band.

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	ARB/39A4/1

4 800-5 570 MHz

| ADD ARB/39A4/2 Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 030-5 150 AERONAUTICAL RADIONAVIGATION 5.367 5.444 5.444A ADD 5.444B |  |  |

5.444B	Additional allocation:  the band 5 091-5 150 MHz is also allocated to the mobile service limited to aeronautical mobile telemetry applications for air-to-ground flight testing. Such use shall not cause harmful interference to nor claim protection from the aeronautical mobile (R) service. The requirements of the aeronautical mobile (R) service shall take precedence over the use of this band by the mobile, including aeronautical mobile, service.

Reasons:	This allocation is necessary for the implementation of AMS in the band 5 091-5 150 MHz. The footnote is to ensure that the allocation to the aeronautical mobile service (AMS) is limited to telemetry for air-to-ground flight testing purposes, without precluding the use of the band by other co-primary services.

SUP	ARB/39A4/3

RESOLUTION  230  (WRC-03)

Consideration of mobile allocations for
wideband aeronautical telemetry
and associated telecommand

Reasons:	To satisfy the requirements of this agenda item.

______________