| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 5 to Document 39-E |
|  | 20 August 2007 |
|  | Original: Arabic |
| Arab States Common Proposals |  |
| PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| Agenda item 1.6 |  |

1.6	to consider additional allocations for the aeronautical mobile (R) service in parts of the bands between 108 MHz and 6 GHz, in accordance with Resolution 414 (WRC-03) and, to study current satellite frequency allocations, that will support the modernization of civil aviation telecommunication systems, taking into account Resolution 415 (WRC-03)

Proposals

Issue A

The Arab States propose:

1)	The removal of the limitation of AM(R)S to systems that transmit navigational information in support of air navigation and surveillance functions as specified in RR No. 5.197A. This implies the following modifications to footnote No. 5.197A and Resolution 413 (WRC-03):

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	ARB/39A5/1

75.2-137.175 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 108-117.975 AERONAUTICAL RADIONAVIGATION 5.197 MOD 5.197A |  |  |

Reasons:	New allocation to the AM(R)S.

MOD	ARB/39A5/2

5.197A	The band 112-117.975 MHz is also allocated to the aeronautical mobile (R) service limited to systems operating in accordance with recognized international aeronautical standards. Such use shall be in accordance with Resolution 413 (Rev.WRC-07).      (WRC-03)

MOD	ARB/39A5/3

RESOLUTION  413  (Rev.WRC-07)

Use of the band 112-117.975 MHz by aeronautical services

The World Radiocommunication Conference (Geneva, 2007),

considering

NOC

a) to e)

f)	the need for the aeronautical community to provide additional services for communications relating to safety and regularity of flight in the band 112-117.975 MHz,

NOC

recognizing

NOC

noting

resolves

1	that any AM(R)S systems operating in the band 112-117.975 MHz shall not cause harmful interference to, nor claim protection from aeronautical radionavigation service systems operating in accordance with international aeronautical standards;

2	that any aeronautical mobile (R) service systems planned to operate in the frequency band 112-117.975 MHz shall, as a minimum, meet the FM broadcasting immunity requirements contained in Annex 10 of the ICAO Convention on International Civil Aviation for existing aeronautical radionavigation systems operating in this frequency band;

3	that aeronautical mobile (R) service systems operating in the band 112-117.975 MHz shall place no additional constraints on the broadcasting service or cause harmful interference to stations operating in the bands allocated to the broadcasting service in the frequency band 87-108 MHz and No. 5.43 does not apply to systems identified in recognizing d);

4	that frequencies below 112 MHz shall not be used for aeronautical mobile (R) service systems excluding the ICAO systems identified in recognizing d) until all potential compatibility issues with the lower adjacent frequency band 87-108 MHz have been resolved,

NOC

invites ITU-R

NOC

instructs the Secretary-General

2)	The addition of a new allocation to the AM(R)S in the band 5 091-5 150 MHz to satisfy airport surface telecommunication requirements. This implies adding a new footnote that defines the use by the AM(R)S and compatibility with the FSS (Earth-to-space):

MOD	ARB/39A5/4

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

4 800-5 570 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 030-5 150 AERONAUTICAL RADIONAVIGATION 5.367 5.444 5.444A ADD 5.444C |  |  |

ADD	ARB/39A5/5

5.444C 	The band 5 091-5 150 MHz is also allocated to the aeronautical mobile (R) service on a primary basis, limited to surface applications at airports by systems operating in accordance with recognized international aeronautical standards.

Reasons:	To add the aeronautical radionavigation service in the band 5 091-5 150 MHz.

Issue C

The Arab States propose to complete the studies in question before any usage of the band 5 091-5 150 MHz for aeronautical security purposes in the aeronautical mobile service.

Issue D

The Arab States propose to suppress Resolution 415 (WRC-03) as there is no need for such a resolution in order to carry out studies concerning the support of civil aviation telecommunication systems.

SUP	ARB/39A5/6

RESOLUTION  415  (WRC-03)

Study of current satellite frequency allocations that will
support the modernization of civil aviation
telecommunication systems

_______________