| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 14 to Document 10-E |
|  | 31 August 2007 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| Part 14 |  |
| Agenda item 1.14 |  |

1.14	to review the operational procedures and requirements of the Global Maritime Distress and Safety System (GMDSS) and other related provisions of the Radio Regulations, taking into account Resolutions 331 (Rev.WRC-03) and 342 (Rev.WRC-2000) and the continued transition to the GMDSS, the experience since its introduction, and the needs of all classes of ships

Introduction

As a result of the review the operational and procedural requirements of the GMDSS, Europe proposes the following amendments:

Appendix 13 contains maritime distress and safety procedures and requirements for the “old” distress and safety system which was intended to be replaced by the GMDSS. To further promote the use of GMDSS Appendix 13 is proposed for suppression. Those elements which need to be retained are proposed to be incorporated in the provisions of Chapter VII.

Based on the experience gained, amendments of the communication procedures of Chapter VII are proposed.

As a consequence of the deletion of Appendix 13 all references to this Appendix in all parts of the Radio Regulations need to be deleted.

The proposed amendments to Appendix 16 align the provisions of this Appendix to the requirements of the modernization of maritime radiocommunication with the transition to the GMDSS.

Appendix 18 requires revision, in order to provide more capacity to meet the increasing communication demand and respond to the needs of data communications.

The intention of the proposals is to reorganize the use of the frequency band of Appendix 18 more efficiently and to provide channels for new technologies by allowing the use of 12.5 kHz channel spacing. Also the proposed extension of the use of footnote m) (splitting of duplex channels into simplex channels) will provide more capacity.

Appendix 19 contains the technical characteristics of emergency position indicating radio beacons operating on the carrier frequency 2 182 kHz. This technique has been replaced by other means using other bands, therefore the Appendix is proposed to be suppressed.

Based upon Agenda item 1.14 for WRC-07, and with respect to the ongoing discussion within ITU-R on the future of the ITU Service Publications for the maritime mobile service within Appendix 16 (see CPM Report Chapter 5, page 78), specific revisions of content, format and on structure are recommended. Therefore the consequential changes to Article 20 are proposed.

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/10A14/1

200-495 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 415-435 MARITIME MOBILE MOD 5.79 AERONAUTICAL RADIONAVIGATION 5.72 | 415-495 MARITIME MOBILE MOD 5.79 (MOD) 5.79A Aeronautical radionavigation 5.80 |  |
| 435-495 MARITIME MOBILE MOD 5.79 (MOD) 5.79A Aeronautical radionavigation |  |  |
| 5.72 MOD 5.82 | 5.77 5.78 MOD 5.82 |  |

MOD	EUR/10A14/2

495-1 800 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 495-505 MOBILE ADD 5.79 ADD 5.A14 |  |  |
| 505-526.5 MARITIME MOBILE MOD 5.79 (MOD) 5.79A MOD 5.84 AERONAUTICAL RADIONAVIGATION | 505-510 MARITIME MOBILE MOD 5.79 | 505-526.5 MARITIME MOBILE MOD 5.79 (MOD) 5.79A MOD 5.84 AERONAUTICAL RADIONAVIGATION Aeronautical mobile |
|  | 510-525 MOBILE (MOD) 5.79A MOD 5.84 AERONAUTICAL RADIONAVIGATION | Land mobile |
| 5.72 |  |  |

MOD	EUR/10A14/3

5.79	The band 415-526.5 kHz (415-510 kHz in Region 2) is designated for radiotelegraphy transmissions in the maritime mobile service.

Reasons: 	By widening the frequency range in this footnote administrations gain more flexibility to use the band 495-505 kHz for other purposes than Morse telegraphy, e.g. Navtex.

(MOD)	EUR/10A14/4

5.79A	When establishing coast stations in the NAVTEX service on the frequencies 490 kHz, 518 kHz and 4 209.5 kHz, administrations are strongly recommended to coordinate the operating characteristics in accordance with the procedures of the International Maritime Organization (IMO) (see Resolution 339 (Rev.WRC-03)).      (WRC-07)

MOD	EUR/10A14/5

5.82	In the maritime mobile service, the frequency 490 kHz is- to be used exclusively for the transmission by coast stations of navigational and meteorological warnings and urgent information to ships, by means of narrow-band direct-printing telegraphy. The conditions for use of the frequency 490 kHz are prescribed in Articles 31 and 52. In using the band  415-495 kHz for the aeronautical radionavigation service, administrations are requested to ensure that no harmful interference is caused to the frequency 490 kHz.     (WRC-07)

SUP	EUR/10A14/6

5.83

ADD	EUR/10A14/7

5.A14	Administrations authorizing the use of frequencies in the band 495-505 kHz by services other than maritime mobile shall ensure that no harmful interference is caused to the maritime mobile service in this band or to the allocated services in the adjacent bands, noting in particular the conditions of use of the frequencies 490 kHz and 518 kHz, as prescribed in Articles 31 and 52.

Reasons:	By widening the frequency range in this footnote administrations gain more flexibility to use this frequency band for other purposes than Morse telegraphy.

MOD	EUR/10A14/8

5.84	The conditions for the use of the frequency 518 kHz by the maritime mobile service are prescribed in Articles 31 and 52.     (WRC-07)

MOD	EUR/10A14/9

1 800-2 194 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 173.5-2 190.5 MOBILE (distress and calling) MOD 5.108 5.109 5.110 MOD 5.111 |  |  |

MOD	EUR/10A14/10

5.108	The carrier frequency 2 182 kHz is an international distress and calling frequency for radiotelephony. The conditions for the use of the band 2 173.5-2 190.5 kHz are prescribed in Articles 31 and 52.

MOD	EUR/10A14/11

2 194-3 230 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 2 850-3 025 AERONAUTICAL MOBILE (R) MOD 5.111 MOD 5.115 |  |  |

MOD	EUR/10A14/12

3 230-5 003 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 4 063-4 438 MARITIME MOBILE (MOD) 5.79A 5.109 5.110 MOD 5.130 5.131 5.132 5.128 5.129 |  |  |

MOD	EUR/10A14/13

5 003-7 450 kHz

| MOD EUR/10A14/14 Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 5 480-5 680 AERONAUTICAL MOBILE (R) MOD 5.111 MOD 5.115 |  |  |
| 5 680-5 730 AERONAUTICAL MOBILE (OR) MOD 5.111 MOD 5.115 |  |  |
| ... |  |  |
| 6 200-6 525 MARITIME MOBILE 5.109 5.110 MOD 5.130 5.132 5.137 |  |  |

7 450-13 360 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 8 195-8 815 MARITIME MOBILE 5.109 5.110 5.132 MOD 5.145 MOD 5.111 |  |  |
| ... |  |  |
| 9 995-10 003 STANDARD FREQUENCY AND TIME SIGNAL (10 000 kHz) MOD 5.111 |  |  |
| 10 003-10 005 STANDARD FREQUENCY AND TIME SIGNAL Space research MOD 5.111 |  |  |
| 10 005-10 100 AERONAUTICAL MOBILE (R) MOD 5.111 |  |  |
| ... |  |  |
| 12 230-13 200 MARITIME MOBILE 5.109 5.110 5.132 MOD 5.145 |  |  |

MOD	EUR/10A14/15

13 360-18 030 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 14 990-15 005 STANDARD FREQUENCY AND TIME SIGNAL (15 000 kHz) MOD 5.111 |  |  |
| ... |  |  |
| 16 360-17 410 MARITIME MOBILE 5.109 5.110 5.132 MOD 5.145 |  |  |

MOD	EUR/10A14/16

18 030-23 350 kHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 19 990-19 995 STANDARD FREQUENCY AND TIME SIGNAL Space research MOD 5.111 |  |  |
| 19 995-20 010 STANDARD FREQUENCY AND TIME SIGNAL (20 000 kHz) MOD 5.111 |  |  |

MOD	EUR/10A14/17

75.2-137.175 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 117.975-137 AERONAUTICAL MOBILE (R) MOD 5.111 5.198 MOD 5.200 5.201 5.202 5.203 5.203A 5.203B |  |  |

MOD	EUR/10A14/18

148-223 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 150.05-153 FIXED MOBILE except aeronautical mobile RADIO ASTRONOMY 5.149 | 150.05-156.4875 FIXED MOBILE |  |
| 153-154 FIXED MOBILE except aeronautical mobile (R) Meteorological Aids |  |  |
| 154-156.4875 FIXED MOBILE except aeronautical mobile (R) |  |  |
| MOD 5.226 | 5.225 MOD 5.226 |  |
| 156.4875-156.5625 MARITIME MOBILE (distress and calling via DSC) | 156.4875-156.5625 MARITIME MOBILE (distress and calling via DSC) |  |
| MOD 5.111 MOD 5.226 MOD 5.227 | MOD 5.111 MOD 5.226 MOD 5.227 |  |
| 156.5625-156.7625 FIXED MOBILE except aeronautical mobile (R) | 156.5625-156.7625 FIXED MOBILE |  |
| MOD 5.226 | 5.225 MOD 5.226 |  |
| 156.7625-156.8375 MARITIME MOBILE (distress and calling) MOD 5.111 MOD 5.226 |  |  |
| 156.8375-174 FIXED MOBILE except aeronautical mobile | 156.8375-174 FIXED MOBILE |  |
| MOD 5.226 5.229 ADD 5.B14 | MOD 5.226 5.230 5.231 5.232 ADD 5.B14 |  |

MOD	EUR/10A14/19

220-335.4 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 235-267 FIXED MOBILE MOD 5.111 5.252 5.254 MOD 5.256 5.256A |  |  |

MOD	EUR/10A14/20

5.111	The carrier frequencies 2 182 kHz, 3 023 kHz, 5 680 kHz, 8 364 kHz and the frequencies 121.5 MHz, 156.525 MHz, 156.8 MHz and 243 MHz may also be used, in accordance with the procedures in force for terrestrial radiocommunication services, for search and rescue operations concerning manned space vehicles. The conditions for the use of the frequencies are prescribed in Article 31.

The same applies to the frequencies 10 003 kHz, 14 993 kHz and 19 993 kHz, but in each of these cases emissions must be confined in a band of  3 kHz about the frequency.

MOD	EUR/10A14/21

5.115	The carrier (reference) frequencies 3 023 kHz and 5 680 kHz may also be used, in accordance with Article 31 by stations of the maritime mobile service engaged in coordinated search and rescue operations.

MOD	EUR/10A14/22

5.130	The conditions for the use of the carrier frequencies 4 125 kHz and 6 215 kHz are prescribed in Articles 31 and 52.

MOD	EUR/10A14/23

5.145	The conditions for the use of the carrier frequencies 8 291 kHz, 12 290 kHz and 16 420 kHz are prescribed in Articles 31 and 52.

SUP	EUR/10A14/24

5.199

MOD	EUR/10A14/25

5.200	In the band 117.975-136 MHz, the frequency 121.5 MHz is the aeronautical emergency frequency and, where required, the frequency 123.1 MHz is the aeronautical frequency auxiliary to 121.5 MHz. Mobile stations of the maritime mobile service may communicate on these frequencies under the conditions laid down in Article 31 for distress and safety purposes with stations of the aeronautical mobile service.

MOD	EUR/10A14/26

5.226	The frequency 156.8 MHz is the international distress, safety and calling frequency for the maritime mobile VHF radiotelephone service. The conditions for the use of this frequency are contained in Article 31 and Appendix 18.

The frequency 156.525 MHz is the international distress, safety and calling frequency for the maritime mobile VHF radiotelephone service using digital selective calling (DSC). The conditions for the use of this frequency are contained in Articles 31 and 52, and Appendix 18.

In the bands 156-156.4875 MHz, 156.5625-156.7625 MHz, 156.8375-157.45 MHz, 160.6-160.975 MHz and 161.475-162.05 MHz, each administration shall give priority to the maritime mobile service on only such frequencies as are assigned to stations of the maritime mobile service by the administration (see Articles 31 and 52, and Appendix 18).

Any use of frequencies in these bands by stations of other services to which they are allocated should be avoided in areas where such use might cause harmful interference to the maritime mobile VHF radiocommunication service.

However, the frequency 156.8 MHz and the frequency bands in which priority is given to the maritime mobile service may be used for radiocommunications on inland waterways subject to agreement between interested and affected administrations and taking into account current frequency usage and existing agreements.

MOD	EUR/10A14/27

5.227	Administrations authorizing the use of frequencies in the bands 156.4875-156.5125 MHz and 156.5375-156.5625 MHz by the fixed and land mobile services shall ensure that no harmful interference is caused to the maritime mobile service in the band 156.4875-156.5625 MHz, noting in particular the conditions of use of the frequency 156.525 MHz, as contained in Articles 31 and 52, and Appendix 18.

Reasons: 	Special protection for Channel 70 and Channel 16 which are to be used in cases of maritime distress.

ADD	EUR/10A14/28

5.B14	The bands 161.9625-161.9875 MHz and 162.0125-162.0375 MHz are also used for satellite reception of AIS transmissions from stations in the maritime mobile service.

MOD	EUR/10A14/29

335.4-410 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 406-406.1 MOBILE-SATELLITE (Earth-to-space) MOD 5.266 5.267 |  |  |

MOD	EUR/10A14/30

5.256	The frequency 243 MHz is the frequency in this band for use by survival craft stations and equipment used for survival purposes.

MOD	EUR/10A14/31

5.266	The use of the band 406-406.1 MHz by the mobile-satellite service is limited to low power satellite emergency position-indicating radiobeacons (see also Article 31).

MOD	EUR/10A14/32

410-460 MHz

| MOD EUR/10A14/33 Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 456-459 FIXED MOBILE 5.271 MOD 5.287 5.288 |  |  |

460-890 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 460-470 FIXED MOBILE Meteorological-Satellite (space-to-Earth) MOD 5.287 5.288 5.289 5.290 |  |  |

MOD	EUR/10A14/34

5.287	In the maritime mobile service, the frequencies 457.525 MHz, 457.550 MHz, 457.575 MHz, 467.525 MHz, 467.550 MHz and 467.575 MHz may be used by on-board communication stations. Where needed, equipment designed for 12.5 kHz channel spacing using also the additional frequencies 457.5375 MHz, 457.5625 MHz, 467.5375 MHz and 467.5625 MHz may be introduced for on-board communications. The use of these frequencies in territorial waters may be subject to the national regulations of the administration concerned. The characteristics of the equipment used shall conform to those specified in Recommendation ITU-R M.1174.     (WRC-07)

ARTICLE  15

Interferences

Section I  –  Interference from Radio Stations

MOD	EUR/10A14/35

15.8	§ 4	Special consideration shall be given to avoiding interference on distress and safety frequencies, those related to distress and safety identified in Article 31, and those related to safety and regularity of flight identified in Appendix 27.     (WRC-07)

Section VI  –  Procedure in a case of harmful interference

MOD	EUR/10A14/36

15.28	§ 20	Recognizing that transmissions on distress and safety frequencies and frequencies used for the safety and regularity of flight (see Article 31 and Appendix 27) require absolute international protection and that the elimination of harmful interference to such transmissions is imperative, administrations undertake to act immediately when their attention is drawn to any such harmful interference.     (WRC-07)

ARTICLE  19

Identification of stations

Section III  –  Formation of call signs

MOD	EUR/10A14/37

19.55	§ 24	1)

–	two characters and two letters, or

–	two characters, two letters and one digit (other than the digits 0 or 1), or

–	two characters (provided that the second is a letter) followed by four digits (other than the digits 0 or 1 in cases where they immediately follow a letter), or

–	two characters and one letter followed by four digits (other than the digits 0 or 1 in cases where they immediately follow a letter).

SUP	EUR/10A14/38

19.56

Reasons: 	Due to the developments of maritime radiocommunications a distinction between ship stations employing radiotelegraphy and radiotelephony is no longer required. Therefore 19.55 and 19.56 are proposed to be combined.

MOD	EUR/10A14/39

19.76		4)	Emergency position-indicating radiobeacon stations

When speech transmission is used:

–	the name and/or the call sign of the parent ship to which the radiobeacon belongs.

ARTICLE  20

Service publications     (WRC-03)

Section I  –  Titles and contents of service publications     (WRC-03)

MOD	EUR/10A14/40

20.1	§ 1	The following publications shall be issued by the Secretary-General. As circumstances warrant and in response to individual requests by administrations, the published information shall also be available in appropriate formats and by appropriate means.     (WRC-03)

MOD	EUR/10A14/41

20.5	b)	the frequencies prescribed by these Regulations for common use by certain services;

MOD	EUR/10A14/42

20.7	§ 3	List IV  –  List of Coast and Special Service Stations.

MOD	EUR/10A14/43

20.8	§ 4	List V  –  List of Ship Stations and Assigned Maritime Mobile Service Identities.

SUP	EUR/10A14/44

20.9

SUP	EUR/10A14/45

20.10

Section II  –  Preparation and amendment of service publications     (WRC-03)

MOD	EUR/10A14/46

20.16	§ 12	Administrations shall take all appropriate measures to notify the Bureau immediately of any changes in the operational information contained in Lists IV and V, in view of the importance of this information, particularly with regard to safety. In the case of List V, administrations shall communicate the changes at least once a month. In the case of other publications, administrations shall communicate the changes in the information contained in them as soon as possible.     (WRC-03)

ADD	EUR/10A14/47

20.16A	The names of the administrations who have failed to notify the Bureau of the changes in the operational information contained in Lists IV and V shall be published in these Lists.

The Bureau will periodically request administrations to reconfirm the information published in List IV. If no information has been received by the Bureau for two consecutive editions of the Lists IV and V, unvalidated information shall be deleted.

ARTICLE  30

General provisions

Section I  –  Introduction

MOD	EUR/10A14/48

30.1	§ 1	This Chapter contains the provisions for the operational use of the global maritime distress and safety system (GMDSS), whose functional requirements, system elements and equipment carriage requirements are set forth in the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended. This Chapter also contains provisions for initiating distress, urgency and safety communications by means of radiotelephony on the frequency 156.8 MHz (VHF channel 16).

EUR/10A14/49

Not used.

Section II  –  Maritime provisions

MOD	EUR/10A14/50

30.4	§ 4	The provisions specified in this Chapter are obligatory in the maritime mobile service and the maritime mobile-satellite service for all stations using the frequencies and techniques prescribed for the functions set out herein (see also No. 30.5).

Section III  –  Aeronautical provisions

ADD	EUR/10A14/51

30.11bis	§ 11bis	Aircraft, when conducting search and rescue operations are also permitted to operate DSC equipment on the VHF DSC frequency 156.525 MHz, and AIS equipment on the AIS frequencies 161.975 MHz and 162.025 MHz.

ARTICLE  31

Frequencies for the global maritime distress and safety system (GMDSS)

Section I  –  General

MOD	EUR/10A14/52

31.1	§ 1	The frequencies to be used for the transmission of distress and safety information under the GMDSS are contained in Appendix 15. In addition to the frequencies listed in Appendix 15, ship stations and coast stations should use other appropriate frequencies for the transmission of safety messages and general radiocommunications to and from shore-based radio systems or networks.

MOD	EUR/10A14/53

31.2	§ 2	Any emission causing harmful interference to distress and safety communications on any of the discrete frequencies identified in Appendix 15 is prohibited.

Section III  –  Watchkeeping

MOD	EUR/10A14/54

31.17	§ 8	1)	Ship stations, where so equipped, shall, while at sea, maintain an automatic digital selective calling watch on the appropriate distress and safety calling frequencies in the frequency bands in which they are operating. Ship stations, where so equipped, shall also maintain watch on the appropriate frequencies for the automatic reception of transmissions of meteorological and navigational warnings and other urgent information to ships..

MOD	EUR/10A14/55

31.18		2)	Ship stations complying with the provisions of this Chapter should, where practicable, maintain a watch on the frequency 156.800 MHz (VHF channel 16).

ARTICLE  32

Operational procedures for distress and safety communications in the global
maritime distress and safety system (GMDSS)

Section I  –  General

MOD	EUR/10A14/56

32.1	§ 1	Distress communications rely on the use of terrestrial MF, HF and VHF radiocommunications and communications using satellite techniques. Distress communications shall have absolute priority over all other transmissions.

MOD	EUR/10A14/57

32.2	§ 2	1)	The distress alert (see No. 32.9) shall be sent through a satellite either with absolute priority in general communication channels, on exclusive distress and safety frequencies reserved for satellite EPIRBs in the Earth-to-space direction or on the distress and safety frequencies designated in the MF, HF and VHF bands (see Appendix 15).

MOD	EUR/10A14/58

32.4	§ 3	All stations which receive a distress alert transmitted on the distress and safety frequencies in the MF, HF and VHF bands shall immediately cease any transmission capable of interfering with distress traffic and prepare for subsequent distress traffic.

MOD	EUR/10A14/59

32.5	§ 4	Calls using digital selective calling should use the technical structure and content set forth in the most recent version of Recommendations ITU-R M.493 and ITU-R M.541.

Section II  –  Distress alerting

ADD	EUR/10A14/60

32.10B 	Administrations shall take practicable and necessary steps to ensure the avoidance of false distress alerts, including those transmitted inadvertently.

MOD	EUR/10A14/61

32.13	§ 9	1)	Ship-to-ship distress alerts are used to alert other ships in the vicinity of the ship in distress and are based on the use of digital selective calling in the VHF and MF bands. Additionally, the HF band may be used.

ADD	EUR/10A14/62

32.13A		2)	Ship stations equipped for making use of digital selective calling procedures may transmit a distress call and distress message immediately following the distress alert in order to attract attention from as many ship stations as possible.

ADD	EUR/10A14/63

32.13B		3)	Ship stations not equipped for making use of the digital selective calling procedures shall, where practical, initiate the distress communications by transmitting a radio telephony distress call and message on the frequency 156.8 MHz (VHF channel 16).

ADD	EUR/10A14/64

32.13C	§ 9A	1)	The distress call shall be given in the following form:

–	the distress signal MAYDAY, spoken three times;

–	the words THIS IS;

–	the name of the vessel in distress, spoken three times;

–	the call sign or other identification;

–	the MMSI (if the initial alert has been sent by DSC).

ADD	EUR/10A14/65

32.13D		2)	The distress message which follows the distress call, shall be given in the following form:

–	the distress signal MAYDAY;

–	the name of the vessel in distress;

–	the call sign or other identification;

–	the MMSI (if the initial alert has been sent by DSC);

–	the position given as the latitude and longitude or with respect to a known geographical location;

–	the nature of the distress;

–	the kind of assistance required;

–	any other useful information.

ADD	EUR/10A14/66

32.13E	§ 9B	Digital selective calling procedures use a combination of automated functions and manual intervention to generate the appropriate distress call format as described in the most recent version of Recommendation ITU-R M.541. The distress alert sent by digital selective calling consists of one or more distress alert attempts in which a message format is transmitted identifying the station in distress, giving its last recorded position and, if entered, the nature of the distress. At MF and HF, distress alert attempts may be sent as a single-frequency attempt or a multifrequency attempt on up to six frequencies within one minute. At VHF, only single-frequency call attempts are used. The distress alert will repeat automatically at random intervals, a few minutes apart, until an acknowledgement sent by digital selective calling is received.

MOD	EUR/10A14/67

Not used.

MOD	EUR/10A14/68

B3  –  Transmission of a distress alert relay by a station not itself in distress

MOD	EUR/10A14/69

32.16	§ 11	A station in the mobile or mobile-satellite service which learns that a mobile unit is in distress (for example, by a radio call or by observation) shall initiate and transmit a distress alert relay on behalf of the mobile unit in distress once it has ascertained that any of the following circumstances apply:

MOD	EUR/10A14/70

32.17	a)	on receiving a distress call sent by radiotelephony on the frequency 156.8 MHz (VHF channel 16), which is not acknowledged by a coast station or another vessel within 5 minutes (see also No. 32.29A);

ADD	EUR/10A14/71

32.17A	b)	on receiving a distress alert on an HF channel, which is not acknowledged by a coast station within 5 minutes (see also No. 32.31);

MOD	EUR/10A14/72

32.18	c)	on knowing that the mobile unit in distress is otherwise unable or incapable of participating in distress communications and the master or other person responsible for the mobile unit not in distress considers that further help is necessary.

MOD	EUR/10A14/73

32.19	§ 12	1)	The distress relay on behalf of a mobile unit in distress shall be sent in a form appropriate to the circumstances using either a MAYDAY relay by radiotelephony (see No. 32.19E), an individually addressed distress relay call by digital selective calling (see No. 32.19B) or a distress priority message through a ship earth station.

ADD	EUR/10A14/74

32.19A		2)	A station transmitting a distress alert relay in accordance with Nos. 32.16 to 32.18 shall indicate that it is not itself in distress.

ADD	EUR/10A14/75

32.19B		3)	A distress alert relay sent by digital selective calling should use the call format, as found in the most recent version of Recommendations ITU-R M.493 and ITU-R M.541 and preferably be addressed to an individual coast station or rescue coordination centrenew1.

ADD	EUR/10A14/76

_______________

new1	32.19B.1	Vessels making a distress relay call should ensure that a suitable coast station or rescue coordination centre is informed of any original distress communications.

ADD	EUR/10A14/77

32.19C		4)	However, a ship shall not transmit a distress relay alert to all ships by digital selective calling on the VHF or MF distress frequencies following receipt of a distress alert sent by digital selective calling by the ship in distress.

ADD	EUR/10A14/78

32.19D		5)	When an aural watch is being maintained on shore and reliable ship-to-shore communications can be established by radiotelephony, a distress relay call shall be sent by radiotelephony and addressed to the relevant coast station or rescue coordination centrenew2 on the appropriate frequency.

ADD	EUR/10A14/79

_______________

new2	32.19D.1	Vessels making a distress relay call should ensure that a suitable coast station or rescue coordination centre is informed of any original distress communications.

ADD	EUR/10A14/80

32.19E		6)	The distress relay call sent by radiotelephony shall be given in the following form:

–	the distress signal MAYDAY RELAY, spoken three times;

–	ALL STATIONS or coast station name spoken three times;

–	the words THIS IS;

–	the name of the relaying station, spoken three times;

–	the call sign or other identification of the relaying station;

–	the MMSI (if the initial alert has been sent by DSC) of the relaying station (the vessel not in distress).

ADD	EUR/10A14/81

32.19F		7)	This call shall be followed by a distress message which shall, as far as possible, repeat the informationnew3 contained in the original distress alert.

ADD	EUR/10A14/82

_______________

new3	32.19F.1	If the station in distress cannot be identified, then it will be necessary to originate the distress message as well, using, for example, terms such as “Unidentified trawler” or “Unidentified helicopter” to refer to the mobile unit in distress.

ADD	EUR/10A14/83

32.19G		8)	When no aural watch is being maintained on shore, or there are other difficulties in establishing reliable ship-to-shore communications by radiotelephony, an appropriate coast station or rescue coordination centre may be contacted by sending an individual distress relay call by digital selective calling addressed solely to that station and using the appropriate call formats.

ADD	EUR/10A14/84

32.19H		9)	In the event of continued failure to contact a coast station or rescue coordination centre directly then it may be appropriate to send a MAYDAY relay by radiotelephony addressed to all ships or to all ships in a certain geographical area. See also No. 32.19C.

MOD	EUR/10A14/85

32.21	§ 13	1)	-Acknowledgement of receipt of a distress alert, including a distress alert relay shall be made in the manner appropriate to the method of transmission of the alert and within the time-scale appropriate to the role of the station in receipt of the alert. Acknowledgement by satellite shall be sent immediately.

ADD	EUR/10A14/86

32.21A		2)	When acknowledging receipt of a distress alert sent by digital selective callingnew4, the acknowledgement in the terrestrial services shall be made by digital selective calling, radiotelephony or narrow-band direct-printing telegraphy as appropriate to the circumstances, on the associated distress and safety frequency in the same band in which the distress alert was received, taking due account of the directions given in the most recent versions of Recommendations ITU-R M.493 and ITU-R M.541.

ADD	EUR/10A14/87

_______________

new4	32.21A.1	In order to ensure that no unnecessary delay occurs before the shore-based authorities become aware of a distress incident, the acknowledgement by digital selective calling to a distress alert sent by digital selective calling shall normally only be made by a coast station or a rescue coordination centre, as an acknowledgement by digital selective calling will cancel any further automated repetition of the distress alert using digital selective calling.

ADD	EUR/10A14/88

32.21B	Acknowledgement by digital selective calling in receipt of a distress alert sent by digital selective calling addressed to stations in the maritime mobile services shall be addressed to the same station as the distress alertnew5.

ADD	EUR/10A14/89

_______________

new5	32.21B.1	In order to ensure that no unnecessary delay occurs before the shore-based authorities become aware of a distress incident, the acknowledgement by digital selective calling to a distress alert sent by digital selective calling shall normally only be made by a coast station or rescue coordination centre, as an acknowledgement by digital selective calling will cancel any further automated repetition of the distress alert using digital selective calling.

SUP	EUR/10A14/90

32.22

MOD	EUR/10A14/91

32.23	§ 15	1)	When acknowledging by radiotelephony the receipt of a distress alert from a ship station or a ship earth station, the acknowledgement shall be given in the following form:

–	the distress signal MAYDAY;

–	the name followed by the call sign or the MMSI or other identification of the station sending the distress message;

–	the words THIS IS;

–	the name and call sign or other identification of the station acknowledging receipt;

–	the word RECEIVED;

–	the distress signal MAYDAY.

MOD	EUR/10A14/92

32.24		2)	When acknowledging by narrow-band direct-printing telegraphy the receipt of a distress alert from a ship station, the acknowledgement shall be given in the following form:

–	the distress signal MAYDAY;

–	the call sign or other identification of the station sending the distress alert;

–	the word DE;

–	the call sign or other identification of the station acknowledging receipt of the distress alert;

–	the signal RRR;

–	the distress signal MAYDAY.

SUP	EUR/10A14/93

32.25

MOD	EUR/10A14/94

32.26	§ 17	Coast stations and the appropriate coast earth stations in receipt of distress alerts shall ensure that they are routed as soon as possible to a rescue coordination centre. In addition, receipt of a distress alert is to be acknowledged as soon as possible by a coast station, or by a rescue coordination centre via a coast station or an appropriate coast earth station. A shore-to-ship distress alert relay (see Nos. 32.14 and 32.15) shall also be made when the method of receipt warrants a broadcast alert to shipping or when the circumstances of the distress incident indicate that further help is necessary.

MOD	EUR/10A14/95

32.27	§ 18	A coast station using digital selective calling to acknowledge a distress alert shall transmit the acknowledgement on the distress calling frequency on which the alert was received and should address it to all ships. The acknowledgement shall include the identification of the ship whose distress alert is being acknowledged.

MOD	EUR/10A14/96

32.29		2)	In areas where reliable communications with one or more coast stations are practicable, ship stations in receipt of a distress alert from another vessel should defer acknowledgement for a short interval so that a coast station may acknowledge receipt in the first instance.

ADD	EUR/10A14/97

32.29A		3)	Ship stations in receipt of a distress call sent by radiotelephony on the frequency 156.8 MHz (VHF channel 16) shall, if the call is not acknowledged by a coast station or another vessel within 5 minutes, acknowledge receipt to the vessel in distress and use any means available to relay the distress alert to an appropriate coast station or coast earth station (see also Nos. 32.16 to 32.19F).

MOD	EUR/10A14/98

32.30	§ 20	1)	Ship stations operating in areas where reliable communications with a coast station are not practicable which receive a distress alert from a ship station which is, beyond doubt, in their vicinity, shall, as soon as possible and if appropriately equipped, acknowledge receipt to the vessel in distress and inform a rescue coordination centre through a coast station or coast earth station (see also Nos. 32.16 to 32.19H).

MOD	EUR/10A14/99

32.31		2)	However, in order to avoid making unnecessary or confusing transmissions in response a ship station receiving an HF distress alert which may be at a considerable distance from the incident, shall not acknowledge it but shall observe the provisions of Nos. 32.36 to 32.38, and shall, if the alert is not acknowledged by a coast station within 5 minutes, relay the distress alert, but only to an appropriate coast station or coast earth station (see also Nos. 32.16 to 32.19H).

MOD	EUR/10A14/100

32.32	§ 21	A ship station acknowledging receipt of a distress alert sent by digital selective calling should, in accordance with No. 32.29 or No. 32.30:

MOD	EUR/10A14/101

32.33	a)	in the first instance, acknowledge receipt of the alert by using radiotelephony on the distress and safety traffic frequency in the band used for the alert, taking into account any potential instructions issued by a responding coast station;

ADD	EUR/10A14/102

32.34A	§ 21A	However, unless instructed to do so by a coast station or a rescue coordination centre, a ship station may only send an acknowledgement by digital selective calling in the event that:

a)	no acknowledgement by digital selective calling from a coast station has been observed; and

b)	no other communication by radiotelephony or narrow-band direct-printing telegraphy to or from the vessel in distress has been observed; and

c)	at least 5 minutes have elapsed and the distress alert by digital selective calling has been repeated (see No. 32.21A.1).

Section III  –  Distress traffic

MOD	EUR/10A14/103

32.45	§ 28	1)	The rescue coordination centre responsible for controlling a search and rescue operation shall also coordinate the distress traffic relating to the incident or may appoint another station to do so.

MOD	EUR/10A14/104

32.51	§ 31	When distress traffic has ceased on frequencies which have been used for distress traffic, the station controlling the search and rescue operation shall initiate a message for transmission on these frequencies indicating that distress traffic has finished.

MOD	EUR/10A14/105

32.52	§ 32	1)	In radiotelephony, the message referred to in No. 32.51 consists of:

–	the distress signal MAYDAY;

–	the call “ALL STATIONS”, spoken three times;

–	the words THIS IS;

–	the name of the station sending that message, spoken three times;

–	the call sign or other identification of the station sending the message;

–	the time of handing in of the message;

–	the MMSI (if the initial alert has been sent by DSC), the name and the call sign of the mobile station which was in distress;

–	the words SEELONCE FEENEE pronounced as the French words “silence fini”.

ADD	EUR/10A14/106

32.53A	Cancellation of an inadvertent distress alert

A station transmitting an inadvertent distress alert shall cancel the distress alert.

Immediately cancel the distress alert orally on the associated distress and safety frequency in the same band on which the “distress alert” was transmitted using the following procedure:

–	All Stations   All Stations   All Stations;

–	the words THIS IS;

–	the name of the vessel, spoken three times;

–	the call sign or other identification;

–	the MMSI (if the initial alert has been sent by DSC);

–	PLEASE CANCEL MY FALSE DISTRESS ALERT of time in UTC,

monitor the same band on which the “distress alert” was transmitted and respond to any communications concerning that distress alert as appropriate.

If the initial alert has been sent by DSC, if the DSC equipment is capable, initiate a SELF CANCELLATION.

MOD	EUR/10A14/107

32.63		3)	Locating signals may be transmitted in the following frequency bands:

117.975-136 MHz;

156-174 MHz;

406-406.1 MHz; and

9 200-9 500 MHz.

SUP	EUR/10A14/108

32.64

ARTICLE  33

Operational procedures for urgency and safety communications in
the global maritime distress and safety system (GMDSS)

Section I  –  General

MOD	EUR/10A14/109

33.1	§ 1	1)	Urgency and safety communications include:

ADD	EUR/10A14/110

33.7A		2)	Urgency communications shall have priority over all other communications, except distress.

ADD	EUR/10A14/111

33.7B		3)	Safety communications shall have priority over all other communications, except distress and urgency.

Section II  –  Urgency communications

MOD	EUR/10A14/112

33.8	§ 2	1)	In a terrestrial system, urgency communications consist of an announcement, transmitted using digital selective calling or radiotelephony, followed by the urgency call and message transmitted using radiotelephony or narrow-band direct-printing. The announcement of the urgency message shall be made on one or more of the distress and safety calling frequencies specified in Section I of Article 31 using either digital selective calling and the urgency call format or, when appropriate, radiotelephony procedures and the urgency signal. Calls using digital selective calling should use the technical structure and content set forth in the most recent version of Recommendations ITU-R M.493 and ITU-R M.541.

ADD	EUR/10A14/113

33.8A		2)	Ship stations not equipped for making use of digital selective calling procedures may announce an urgency call and message by transmitting the urgency signal by radiotelephony on the frequency 156.8 MHz (channel 16), while taking into account that other stations outside VHF range may not receive the announcement.

ADD	EUR/10A14/114

33.8B		3)	In the maritime mobile service, urgency communications may be addressed either to all stations or to a particular station. When using digital selective calling techniques, the urgency announcement shall indicate which frequency is to be used to send the subsequent message and, in the case of a message to all stations, shall use the “All Ships” format setting.

ADD	EUR/10A14/115

33.8C		4)	Urgency communications from a coast station may also be directed to a group of vessels or to vessels in a defined geographical area.

MOD	EUR/10A14/116

33.9	§ 3	1)	The urgency signal and message shall be transmitted on one or more of the distress and safety traffic frequencies specified in Section I of Article 31.

ADD	EUR/10A14/117

33.9A		2)	However, in the maritime mobile service, the message shall be transmitted on a working frequency:

a)	in the case of a long message or a medical call; or

b)	in areas of heavy traffic when the message is being repeated.

An indication to this effect shall be included in the announcement.

ADD	EUR/10A14/118

33.9B		3)	In the maritime mobile-satellite service, a separate announcement does not need to be made before sending the urgency message. However, if available, the appropriate network priority access settings should be used for sending the message.

MOD	EUR/10A14/119

33.11	§ 5	1)	The urgency call format and the urgency signal indicate that the calling station has a very urgent message to transmit concerning the safety of a mobile unit or a person.

ADD	EUR/10A14/120

33.11A		2)	Communications concerning medical advice may be preceded by the urgency signal. Mobile stations requiring medical advice may obtain it through any of the land stations shown in the List of Radiodetermination and Special Service Stations.

ADD	EUR/10A14/121

33.11B		3)	Urgency communications to support search and rescue operations need not be preceded by the urgency signal.

MOD	EUR/10A14/122

33.12	§ 6	1)	The urgency call consists of:

–	the urgency signal PAN PAN, spoken three times;

–	the name of the called station or “all stations”, spoken three times;

–	the words THIS IS;

–	the name of the station transmitting the urgency message, spoken three times;

–	the call sign or any other identification;

–	the MMSI (if the initial announcement has been sent by DSC),

followed by the message or followed by the details of the channel to be used for the message in the case where a working channel is to be used.

In radiotelephony, on the selected working frequency, the urgency call and message consists of:

–	the urgency signal PAN PAN, spoken three times;

–	the name of the called station or “all stations”, spoken three times;

–	the words THIS IS;

–	the name of the station transmitting the urgency message, spoken three times;

–	the call sign or any other identification;

–	the MMSI (if the initial announcement has been sent by DSC);

–	the text of the urgency message.

MOD	EUR/10A14/123

33.14	§ 7	1)	The urgency call format or urgency signal shall be sent only on the authority of the person responsible for the ship, aircraft or other vehicle carrying the mobile station or mobile earth station.

ADD	EUR/10A14/124

33.15A	§ 7A	1)	Ship stations in receipt of an urgency announcement or call addressed to all stations shall not acknowledge.

ADD	EUR/10A14/125

33.15B		2)	Ship stations in receipt of an urgency announcement or call of an urgency message shall monitor the frequency or channel indicated for the message for at least five minutes. If, at the end of the five-minute monitoring period, no urgency message has been received, a coast station should, if possible, be notified of the missing message. Thereafter, normal working may be resumed.

ADD	EUR/10A14/126

33.15C		3)	Coast and ship stations which are in communication on frequencies other than those used for the transmission of the urgency signal or the subsequent message may continue their normal work without interruption, provided that the urgency message is not addressed to them nor broadcast to all stations.

MOD	EUR/10A14/127

33.16	§ 8	When an urgency message was transmitted to more than one station and action is no longer required an urgency cancellation shall be sent by the station responsible for its transmission.

The urgency cancellation consists of:

–	the urgency signal PAN PAN, spoken three times;

–	“all stations”, spoken three times;

–	the words THIS IS;

–	the name of the station transmitting the urgency message, spoken three times;

–	the call sign or any other identification;

–	the MMSI (if the initial announcement has been sent by DSC);

–	PLEASE CANCEL MY URGENCY MESSAGE of time in UTC.

Section III  –  Medical transports

MOD	EUR/10A14/128

33.20	§ 11	1)	For the purpose of announcing and identifying medical transports which are protected under the above-mentioned Conventions, the procedure of Section II of this Article is used. The urgency signal shall be followed by the addition of the single word MEDICAL in narrow-band direct-printing and by the addition of the single word MAY-DEE-CAL pronounced as in French “médical”, in radiotelephony.

ADD	EUR/10A14/129

33.20A		2)	When using digital selective calling techniques, the announcement on the appropriate digital selective calling distress and safety frequencies shall always use the following call format:

–	Format Specifier: “ALL SHIPS” on VHF, and geographical area on HF and MF;

–	Category: “URGENCY”;

–	Telecommand: “MEDICAL TRANSPORT”.

ADD	EUR/10A14/130

33.20B		3)	Medical transports may use one or more of the distress and safety traffic frequencies specified in Section I of Article 31 for the purpose of self-identification and to establish communications. As soon as practicable, communications shall be transferred to an appropriate working frequency.

MOD	EUR/10A14/131

33.21	§ 12	The use of the signals described in Nos. 33.20 and 33.20A indicates that the message which follows concerns a protected medical transport. The message shall convey the following data:

SUP	EUR/10A14/132

33.28

SUP	EUR/10A14/133

33.29

Section IV  –  Safety communications

MOD	EUR/10A14/134

33.31	§ 15	1)	In a terrestrial system, safety communications consist of an announcement, transmitted using digital selective calling or radiotelephony, followed by the safety call and message transmitted using radiotelephony or narrow-band direct-printing. The announcement of the safety message shall be made on one or more of the distress and safety calling frequencies specified in Section I of Article 31 using either digital selective calling techniques and the safety call format or radiotelephony procedures and the safety signal.

MOD	EUR/10A14/135

33.31A		2)	However, in order to avoid unnecessary loading of the distress and safety calling frequencies specified for use with digital selective calling techniques:

a)	safety messages transmitted by coast stations in accordance with a predefined timetable should not be announced by digital selective calling techniques;

b)	the radiotelephony procedures for announcing the transmission of a safety message should also be used when the message only concerns vessels sailing in the immediate area.     (WRC-03)

ADD	EUR/10A14/136

33.31B		3)	In addition, ship stations not equipped for making use of digital selective calling procedures may announce a safety message by transmitting the safety signal by radiotelephony. In such cases the announcement shall be made using the frequency 156.8 MHz (VHF channel 16), while taking into account that other stations outside VHF range may not receive the announcement.

ADD	EUR/10A14/137

33.31C		4)	In the maritime mobile service, safety messages shall generally be addressed to all stations. In some cases, however, they may be addressed to a particular station. When using digital selective calling techniques, the safety announcement shall indicate which frequency is to be used to send the subsequent message and, in the case of a message to all stations, shall use the “All Ships” format setting.

MOD	EUR/10A14/138

33.32	§ 16	1)	In the maritime mobile service, the safety message shall, where practicable, be transmitted on a working frequency in the same band(s) as used for the announcement. A suitable indication to this effect shall be made at the end of the announcement. In the case that no other option is practicable, the safety message may be sent by radiotelephony on the frequency 156.8 MHz (VHF channel 16).

ADD	EUR/10A14/139

33.32A		2)	In the maritime mobile-satellite service, a separate announcement does not need to be made before sending the safety message. However, if available, the appropriate network priority access settings should be used for sending the message.

MOD	EUR/10A14/140

33.34	§ 18	1)	The safety call format or the safety signal indicates that the calling station has an important navigational or meteorological warning to transmit.

ADD	EUR/10A14/141

33.34A		2)	Messages from ship stations containing information concerning the presence of cyclones shall be transmitted, with the least possible delay, to other mobile stations in the vicinity and to the appropriate authorities through a coast station, or through a rescue coordination centre via a coast station or an appropriate coast earth station. These transmissions shall be preceded by the safety signal.

ADD	EUR/10A14/142

33.34B		3)	Messages from ship stations containing information on the presence of dangerous ice, dangerous wrecks, or any other imminent danger to marine navigation, shall be transmitted as soon as possible to other ships in the vicinity, and to the appropriate authorities through a coast station, or through a rescue coordination centre via a coast station or an appropriate coast earth station. These transmissions shall be preceded by the safety signal.

MOD	EUR/10A14/143

33.35	§ 19	1)	The complete safety call consists of:

–	the safety signal SÉCURITÉ, spoken three times;

–	the name of the called station or “all stations”, spoken three times;

–	the words THIS IS;

–	the name of the station transmitting the safety message, spoken three times;

–	the call sign or any other identification;

–	the MMSI (if the initial announcement has been sent by DSC),

followed by the safety message or followed by the details of the channel to be used for the message in the case where a working channel is to be used.

In radiotelephony, on the selected working frequency, the safety call and message consists of:

–	the safety signal SÉCURITÉ, spoken three times;

–	the name of the called station or “all stations”, spoken three times;

–	the words THIS IS;

–	the name of the station transmitting the safety message, spoken three times;

–	the call sign or any other identification;

–	the MMSI (if the initial alert has been sent by DSC);

–	the text of the safety message.

ADD	EUR/10A14/144

33.38A	§ 20bis	1)	Ship stations in receipt of a safety announcement using digital selective calling techniques and the “All Ships” format setting, or otherwise addressed to all stations shall not acknowledge.

ADD	EUR/10A14/145

33.38B		2)	Ship stations in receipt of an announcement of a safety message shall monitor the frequency or channel indicated for the message and shall listen until they are satisfied that the message is of no concern to them. They shall not make any transmission likely to interfere with the message.

MOD	EUR/10A14/146

Section V  –  Transmission of maritime safety informationMOD 1

MOD	EUR/10A14/147

_______________

1 	33.V.1	Maritime safety information includes navigation and meteorological warnings, meteorological forecasts and other urgent messages pertaining to safety transmitted from coast stations or coast earth stations.

SUP	EUR/10A14/148

33.39A

SUP	EUR/10A14/149

33.39B

SUP	EUR/10A14/150

33.40

MOD	EUR/10A14/151

Section VII  –  Use of other frequencies for safety

33.53	§ 28	Radiocommunications for safety purposes concerning ship reporting communications, communications relating to the navigation, movements and needs of ships and weather observation messages may be conducted on any appropriate communications frequency, including those used for public correspondence. In terrestrial systems, the bands between 415 kHz and 535 kHz (see Article 52), 1 606.5 kHz and 4 000 kHz (see Article 52), 4 000 kHz and 27 500 kHz (see Appendix 17) and 156 MHz and 174 MHz (see Appendix 18) are used for this function. In the maritime mobile-satellite service, frequencies in the bands 1 530-1 544 MHz and 1 626.5-1 645.5 MHz are used for this function as well as for distress alerting purposes (see No. 32.2).

SUP	EUR/10A14/152

Section VIII  –  Medical advice

SUP	EUR/10A14/152A

33.54

SUP	EUR/10A14/153

33.55

ARTICLE  34

Alerting signals in the global maritime distress and safety system (GMDSS)

Section I  –  Emergency position-indicating radiobeacon (EPIRB) and
satellite EPIRB signals

MOD	EUR/10A14/154

34.1	§ 1	The emergency position-indicating radiobeacon signals shall be in accordance with relevant ITU-R Recommendations-.

Reasons:	As a consequence of the changed requirements for the carriage of EPIRBs in the GMDSS, such as resulting from the decision to discontinue the INMARSAT EPIRB service, it will be clearer to remove all references to frequency bands.

ARTICLE  41

Communications with stations in the maritime services

MOD	EUR/10A14/155

41.1		Stations on board aircraft may communicate, for purposes of distress, and for public correspondence1, with stations of the maritime mobile or maritime mobile-satellite services. For these purposes, they shall conform to the relevant provisions of Chapter VII and Chapter IX, Articles 51 (Section III), 53, 54, 55, 57 and 58 (see also Nos. 4.19, 4.20 and 43.4).

ARTICLE  47

Operator’s certificates

Section I  –  General provisions

MOD	EUR/10A14/156

47.18	§ 5	Each administration may determine the conditions under which personnel holding certificates may be granted certificates specified in Nos. 47.20 to 47.23.

MOD	EUR/10A14/157

Section IV  –  Qualifying service     (WRC-03)

SUP	EUR/10A14/158

_______________

1	47.IV.1

ARTICLE  51

Conditions to be observed in the maritime services

Section I  –  Maritime mobile service

SUP	EUR/10A14/159

51.8 to 51.23

Reasons:	51.8 to 51.23 contain provisions for ship stations using Morse Code. International Morse Code Communications have drawn to an end and therefore these regulations are no longer needed in the RR.

MOD	EUR/10A14/160

51.53	a)	send class J3E emissions on a carrier frequency of 2 182 kHz and receive class J3E emissions on a carrier frequency of 2 182 kHz, except for such apparatus as is referred to in No. 51.56;

MOD	EUR/10A14/161

51.58	§ 23	All ship stations equipped with radiotelephony to work in the authorized bands between 4 000 kHz and 27 500 kHz and which do not comply with the provisions of Chapter VII should be able to send and receive on the carrier frequencies 4 125 kHz and 6 215 kHz. However, all ship stations which comply with the provisions of Chapter VII shall be able to send and receive on the carrier frequencies designated in Article 31 for distress and safety traffic by radiotelephony for the frequency bands in which they are operating.

Section III  –  Stations on board aircraft communicating with stations of
the maritime mobile service and the maritime mobile-satellite service

MOD	EUR/10A14/162

51.79		2)	The frequency 156.3 MHz may be used by stations on board aircraft for safety purposes. It may also be used for communication between ship stations and stations on board aircraft engaged in coordinated search and rescue operations (see Appendix 15).

MOD	EUR/10A14/163

51.80		3)	The frequency 156.8 MHz may be used by stations on board aircraft for safety purposes only (see Appendix 15).

ARTICLE  52

Special rules relating to the use of frequencies

SUP	EUR/10A14/164

Section II  –  Use of frequencies for Morse radiotelegraphy

SUP	EUR/10A14/165

52.16 to 52.93

Reasons: 	Article 52 contains special rules relating to the use of frequencies. Significant part of these rules relate to Morse Code communications. International Morse Code communications have drawn to an end and therefore these regulations are no longer needed in the Radio Regulations.

Section III  –  Use of frequencies for narrow-band direct-printing telegraphy

MOD	EUR/10A14/166

52.101		2)	Narrow-band direct-printing telegraphy is forbidden in the band 2 170-2 194 kHz except as provided for in Appendix 15.

Section VI  –   Use of frequencies for radiotelephony

MOD	EUR/10A14/167

52.183	§ 86	1)	Unless otherwise specified in the present Regulations (see Nos. 51.53, 52.188, 52.189 and 52.199), the class of emission to be used in the bands between 1 606.5 kHz and 4 000 kHz shall be J3E.     (WRC-03)

MOD	EUR/10A14/168

52.189	§ 87	1)	The frequency 2 182 kHz2 is an international distress frequency for radiotelephony.

MOD	EUR/10A14/169

52.209		2)	The carrier frequencies 2 045 kHz and 2 048 kHz shall not be used for working between stations of the same nationality*.

MOD	EUR/10A14/170

_______________

4 	52.221.2	The carrier frequencies 4 125 kHz and 6 215 kHz are also authorized for common use by coast and ship stations for single-sideband radiotelephony on a simplex basis for call and reply purposes, provided that the peak envelope power of such stations does not exceed 1 kW. The use of these frequencies for working purposes is not permitted (see also No. 52.221.1).

MOD	EUR/10A14/171

52.231	§ 101	1)	The frequency 156.8 MHz is the international frequency for distress traffic and for calling by radiotelephony when using frequencies in the authorized bands between 156 MHz and 174 MHz. The class of emission to be used for radiotelephony on the frequency 156.8 MHz shall be G3E (as specified in Recommendation ITU-R M.489-2).     (WRC-03)

ADD	EUR/10A14/172

52.241A		10)	The frequency 156.525 MHz is the international distress, safety and calling frequency for the maritime mobile VHF radiotelephone service using digital selective calling (DSC) when using frequencies in the authorized bands between 156 MHz and 174 MHz.

ADD	EUR/10A14/173

52.241B		11)	All emissions in the band 156.4875-156.5625 MHz capable of causing harmful interference to the authorized transmissions of stations of the maritime mobile service on 156.525 MHz are forbidden.

ADD	EUR/10A14/174

52. 241C		12)	To facilitate the reception of distress calls and distress traffic, all transmissions on 156.525 MHz shall be kept to a minimum.

MOD	EUR/10A14/175

52.242	§ 102	1)	A coast station open to the international public correspondence service should, during its hours of service, maintain watch on its receiving frequency or frequencies indicated in the List of Coast Stations.

ARTICLE  57

Radiotelephony

MOD	EUR/10A14/176

57.1	§ 1	The procedure detailed in Recommendation ITU-R M.1171 shall be applicable to radiotelephone stations, except in cases of distress, urgency or safety.-

MOD	EUR/10A14/177

57.8	§ 4	Calling, and signals preparatory to traffic, shall not exceed one minute when made on the carrier frequency 2 182 kHz or on 156.8 MHz, except in cases of distress, urgency or safety.

SUP	EUR/10A14/178

APPENDIX  13  (Rev.WRC-03)*

Distress and safety communications (non-GMDSS)

(See Article 30)

Reasons: 	Appendix 13 contains the maritime distress and safety procedures for the old distress and safety system (non-GMDSS). Appendix 13 is largely obsolete and should be suppressed. Some of the procedures in Appendix 13, however, need to be retained in other parts of the Radio Regulations. This requires consequent changes to Articles 5, 15, 30, 31, 32, 33, 34, 51, 52 and 57 as well as to Appendices 14, 15 and 17.

MOD	EUR/10A14/179

APPENDIX  14

Phonetic alphabet and figure code

(See Articles 30 and 57)

Note: The remaining part of Appendix 14 is unchanged.

MOD	EUR/10A14/180

APPENDIX  15  (Rev.WRC-07)

Frequencies for distress and safety communications for the Global
Maritime Distress and Safety System (GMDSS)

(See Article 31)

The frequencies for distress and safety communications for the GMDSS are given in Tables 15-1 and 15-2 for frequencies below and above 30 MHz, respectively.

TABLE  15-1

Frequencies below 30 MHz

| Frequency (kHz) | Description of usage | Notes |
| --- | --- | --- |
| 490 | MSI | The frequency 490 kHz is used exclusively for maritime safety information (MSI). (WRC-03) |
| 518 | MSI | The frequency 518 kHz is used exclusively by the international NAVTEX system. |
| *2 174.5 | NBDP-COM |  |
| *2 182 | RTP-COM | The frequency 2 182 kHz uses class of emission J3E. See also No. 52.190. |
| *2 187.5 | DSC |  |
| 3 023 | AERO-SAR | The aeronautical carrier (reference) frequencies 3 023 kHz and 5 680 kHz may be used for intercommunication between mobile stations engaged in coordinated search and rescue operations, and for communication between these stations and participating land stations, in accordance with the provisions of Appendix 27 (see Nos. 5.111 and 5.115). |
| *4 125 | RTP-COM | See also No. 52.221. The carrier frequency 4 125 kHz may be used by aircraft stations to communicate with stations of the maritime mobile service for distress and safety purposes, including search and rescue (see No. 30.11). |
| *4 177.5 | NBDP-COM |  |
| *4 207.5 | DSC |  |
| 4 209.5 | MSI | The frequency 4 209.5 kHz is exclusively used for NAVTEX-type transmissions (see Resolution 339 (Rev.WRC-03)). |
| 4 210 | MSI-HF |  |
| 5 680 | AERO-SAR | See note under 3 023 kHz above. |
| *6 215 | RTP-COM | See also No. 52.221. |
| *6 268 | NBDP-COM |  |
| *6 312 | DSC |  |

TABLE  15-1 (end )

| Frequency (kHz) | Description of usage | Notes |
| --- | --- | --- |
| 6 314 | MSI-HF |  |
| *8 291 | RTP-COM |  |
| *8 376.5 | NBDP-COM |  |
| *8 414.5 | DSC |  |
| 8 416.5 | MSI-HF |  |
| *12 290 | RTP-COM |  |
| *12 520 | NBDP-COM |  |
| *12 577 | DSC |  |
| 12 579 | MSI-HF |  |
| *16 420 | RTP-COM |  |
| *16 695 | NBDP-COM |  |
| *16 804.5 | DSC |  |
| 16 806.5 | MSI-HF |  |
| 19 680.5 | MSI-HF |  |
| 22 376 | MSI-HF |  |
| 26 100.5 | MSI-HF |  |
| Legend: AERO-SAR These aeronautical carrier (reference) frequencies may be used for distress and safety purposes by mobile stations engaged in coordinated search and rescue operations. DSC These frequencies are used exclusively for distress and safety calls using digital selective calling in accordance with No. 32.5 (see Nos. 32.9, 33.11 and 33.34). MSI In the maritime mobile service, these frequencies are used exclusively for the transmission of maritime safety information (MSI) (including meteorological and navigational warnings and urgent information) by coast stations to ships, by means of narrow-band direct-printing telegraphy. MSI-HF In the maritime mobile service, these frequencies are used exclusively for the transmission of high seas MSI by coast stations to ships, by means of narrow-band direct-printing telegraphy. NBDP-COM These frequencies are used exclusively for distress and safety communications (traffic) using narrow-band direct-printing telegraphy. RTP-COM These carrier frequencies are used for distress and safety communications (traffic) by radiotelephony. * Except as provided in these Regulations, any emission capable of causing harmful interference to distress, alarm, urgency or safety communications on the frequencies denoted by an asterisk (*) is prohibited. Any emission causing harmful interference to distress and safety communications on any of the discrete frequencies identified in this Appendix is prohibited. |  |  |

TABLE  15-2

Frequencies above 30 MHz (VHF/UHF)

| Frequency (MHz) | Description of usage | Notes |
| --- | --- | --- |
| *121.5 | AERO-SAR | The aeronautical emergency frequency 121.5 MHz is used for the purposes of distress and urgency for radiotelephony by stations of the aeronautical mobile service using frequencies in the band between 117.975 MHz and 137 MHz. This frequency may also be used for these purposes by survival craft stations. Emergency position-indicating radio beacons use the frequency 121.5 MHz as indicated in Recommendation ITU-R M.690-1. Mobile stations of the maritime mobile service may communicate with stations of the aeronautical mobile service on the aeronautical emergency frequency 121.5 MHz for the purposes of distress and urgency only, and on the aeronautical auxiliary frequency 123.1 MHz for coordinated search and rescue operations, using class A3E emissions for both frequencies (see also Nos. 5.111 and 5.200). They shall then comply with any special arrangement between governments concerned by which the aeronautical mobile service is regulated. |
| 123.1 | AERO-SAR | The aeronautical auxiliary frequency 123.1 MHz, which is auxiliary to the aeronautical emergency frequency 121.5 MHz, is for use by stations of the aeronautical mobile service and by other mobile and land stations engaged in coordinated search and rescue operations (see also No. 5.200). Mobile stations of the maritime mobile service may communicate with stations of the aeronautical mobile service on the aeronautical emergency frequency 121.5 MHz for the purposes of distress and urgency only, and on the aeronautical auxiliary frequency 123.1 MHz for coordinated search and rescue operations, using class A3E emissions for both frequencies (see also Nos. 5.111 and 5.200). They shall then comply with any special arrangement between governments concerned by which the aeronautical mobile service is regulated. |
| 156.3 | VHF-CH06 | The frequency 156.3 MHz may be used for communication between ship stations and aircraft stations engaged in coordinated search and rescue operations. It may also be used by aircraft stations to communicate with ship stations for other safety purposes (see also Note f ) in Appendix 18). |
| *156.525 | VHF-CH70 | The frequency 156.525 MHz is used in the maritime mobile service for distress and safety calls using digital selective calling (see also Nos. 4.9, 5.227, 30.2 and 30.3). |
| 156.650 | VHF-CH13 | The frequency 156.650 MHz is used for ship-to-ship communications relating to the safety of navigation in accordance with Note k) in Appendix 18. |
| *156.8 | VHF-CH16 | The frequency 156.8 MHz is used for distress and safety communications by radiotelephony. Additionally, the frequency 156.8 MHz may be used by aircraft stations for safety purposes only. |

TABLE  15-2 (end )

| Frequency (MHz) | Description of usage | Notes |
| --- | --- | --- |
| *406-406.1 | 406-EPIRB | This frequency band is used exclusively by satellite emergency position-indicating radio beacons in the Earth-to-space direction (see No. 5.266). |
| 1 530-1 544 | SAT-COM | In addition to its availability for routine non-safety purposes, the band 1 530-1 544 MHz is used for distress and safety purposes in the space-to-Earth direction in the maritime mobile-satellite service. GMDSS distress, urgency and safety communications have priority in this band (see No. 5.353A). |
| *1 544-1 545 | D&S-OPS | Use of the band 1 544-1 545 MHz (space-to-Earth) is limited to distress and safety operations (see No. 5.356). |
| 1 626.5-1 645.5 | SAT-COM | In addition to its availability for routine non-safety purposes, the band 1 626.5-1 645.5 MHz is used for distress and safety purposes in the Earth-to-space direction in the maritime mobile-satellite service. GMDSS distress, urgency and safety communications have priority in this band (see No. 5.353A). |
| *1 645.5-1 646.5 | D&S-OPS | Use of the band 1 645.5-1 646.5 MHz (Earth-to-space) is limited to distress and safety operations (see No. 5.375). |
| 9 200-9 500 | SARTS | This frequency band is used by radar transponders to facilitate search and rescue. |
| Legend: AERO-SAR These aeronautical carrier (reference) frequencies may be used for distress and safety purposes by mobile stations engaged in coordinated search and rescue operations. D&S-OPS The use of these bands is limited to distress and safety operations of satellite emergency position-indicating radio beacons (EPIRBs). SAT-COM These frequency bands are available for distress and safety purposes in the maritime mobile-satellite service (see Notes). VHF-CH# These VHF frequencies are used for distress and safety purposes. The channel number (CH#) refers to the VHF channel as listed in Appendix 18, which should also be consulted. * Except as provided in these Regulations, any emission capable of causing harmful interference to distress, alarm, urgency or safety communications on the frequencies denoted by an asterisk (*) is prohibited. Any emission causing harmful interference to distress and safety communications on any of the discrete frequencies identified in this Appendix is prohibited. |  |  |

MOD	EUR/10A14/181

APPENDIX  16

Documents with which stations on board ships
and aircraft shall be provided

(See Articles 42 and 51)

Section I  –  Ship stations for which a GMDSS installation is required by international agreement

These stations shall be provided with:

1	the licence prescribed by Article 18;

2	certificates of the operator or operators;

3	a log in which the following are recorded as they occur, together with the time of the occurrence, unless administrations have adopted other arrangements for recording all information which the log should contain:

a)	a summary of communications relating to distress, urgency and safety traffic;

b)	a reference to important service incidents;

c)	if the ship’s rules permit, the position of the ship at least once a day;

4	ITU Service Publications, in either printed or electronic format, containing a list of MMSI stations and other operational information in the maritime mobile service (see Article 20);

5	ITU Service Publications, in either printed or electronic format, showing the details of coast stations and coast earth stations with which communications are likely to be conducted and a list of coast stations and coast earth stations providing navigational and meteorological warnings and other urgent information for ships (see Article 20);

6	the relevant rules and procedures of radiocommunications, e.g. Manual for use by the maritime mobile and maritime mobile-satellite services (paper or electronic format) (see Article 20).

NOTE – Administrations may, under appropriate circumstances, exempt ships from the carriage of the documents mentioned in items 4 to 6 above or where equivalent publications with the same and similarly updated content exist, notify the changed carriage requirement to the Bureau which will make this publicly available.

Section II  –  Other ship stations for which a radiotelephone installation is required

These stations shall be provided with:

1	the licence prescribed by Article 18;

2	certificates of the operator or operators;

3	a log or other arrangements which the administration may have adopted for that purpose, in which a summary of communications related to distress, urgency and safety traffic shall be recorded together with the time of their occurrence;

4	ITU Service Publications, in either printed or electronic format, showing the details of coast stations with which communications are likely to be conducted and a list of coast stations and coast earth stations providing navigational and meteorological warnings and other urgent information for ships (see Article 20);

5	-the relevant rules and procedures of radiocommunications, e.g. Manual for use by the maritime mobile and maritime mobile-satellite services (paper or electronic format) (see Article 20).

NOTE – Administrations may, under appropriate circumstances, exempt ships from the carriage of the documents mentioned in items 4 and 5 above or where equivalent publications with the same and similarly updated content exist, notify the changed carriage requirement to the Bureau which will make this publicly available.

Section III  –  Other ship stations

These stations shall be provided with:

1	the documents mentioned in items 1 and 2 of Section II;

2	the documents mentioned in items 4 and 5 of Section II, in accordance with the requirements of the administrations concerned.

NOTE – Administrations may, under appropriate circumstances, exempt ships from the carriage of the documents mentioned in item 2 or where equivalent publications with the same and similarly updated content exist, notify the changed carriage requirement to the Bureau which will make this publicly available.

Section IV  –  Stations on board aircraft

These stations shall be provided with:

1	the documents mentioned in items 1 and 2 of Section I;

2	a log, unless administrations have adopted other arrangements for recording all information which the log should contain;

3	those published documents, in either printed or electronic formats, containing official information relating to stations which the aircraft station may use for the execution of its service.

APPENDIX  17  (Rev.WRC-03)

Frequencies and channelling arrangements in the
high-frequency bands for the maritime mobile service

(See Article 52)

PART  A  –  Table of subdivided bands     (WRC-03)

Note by the secretariat: The following suppression/modification relates to the notes to the table in this part.

SUP	EUR/10A14/182

h)

MOD	EUR/10A14/183

i)	For the use of the carrier frequencies 4 125 kHz, 6 215 kHz, 8 291 kHz, 12 290 kHz and 16 420 kHz in these sub-bands by ship and coast stations for distress and safety purposes, by single-sideband radiotelephony, see Article 31.

PART  B  –  Channelling arrangements

Section I  –  Radiotelephony

MOD	EUR/10A14/184

5A	For the use of the carrier frequencies:

4 125 kHz (Channel No. 421);

6 215 kHz (Channel No. 606);

8 291 kHz (Channel No. 833);

12 290 kHz (Channel No. 1221);

16 420 kHz (Channel No. 1621);

in Sub-Section A, by coast and ship stations for distress and safety purposes, see Article 31.

Reasons: 	The proposed amendments to Appendix 17 are consequential to the suppression of Appendix 13. This proposals should be seen in addition to the proposals of the ECP in response Resolution 351 under Agenda item 1.13.

APPENDIX  18  (WRC-07)

Table of transmitting frequencies in the VHF
maritime mobile band

(See Article 52)

MOD	EUR/10A14/185

NOTE A – For assistance in understanding the Table, see Notes a) to q) below.     -

ADD	EUR/10A14/186

NOTE B – The Table below defines the channel numbering for conventional maritime VHF based on 25 kHz channel spacing and use of several duplex channels, but allows also the use of 12.5 kHz channel spacing. The channel numbering for 12.5 kHz channels and the conversion of two-frequency channels for single-frequency operation shall be in accordance with Recommendation ITU-R M.1084-4 Annex 4, Tables 1 and 3.

MOD	EUR/10A14/187

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ship stations | Coast stations |  | Single frequency | Two frequency |  |
| 60 | m), o) | 156.025 | 160.625 |  |  | x | x |
| 01 | m), o) | 156.050 | 160.650 |  |  | x | x |
| 61 | m), o) | 156.075 | 160.675 |  | x | x | x |
| 02 | m), o) | 156.100 | 160.700 |  | x | x | x |
| 62 | m), o) | 156.125 | 160.725 |  | x | x | x |
| 03 | m), o) | 156.150 | 160.750 |  | x | x | x |
| 63 | m), o) | 156.175 | 160.775 |  | x | x | x |
| 04 | m), o) | 156.200 | 160.800 |  | x | x | x |
| 64 | m), o) | 156.225 | 160.825 |  | x | x | x |
| 05 | m), o) | 156.250 | 160.850 |  | x | x | x |
| 65 | m), o) | 156.275 | 160.875 |  | x | x | x |
| 06 | f) | 156.300 |  | x |  |  |  |
| 66 | m), o) | 156.325 | 160.925 |  |  | x | x |
| 07 | m), o) | 156.350 | 160.950 |  |  | x | x |
| 67 | h) | 156.375 | 156.375 | x | x |  |  |
| 08 |  | 156.400 |  | x |  |  |  |
| 68 |  | 156.425 | 156.425 |  | x |  |  |
| 09 | i) | 156.450 | 156.450 | x | x |  |  |
| 69 |  | 156.475 | 156.475 | x | x |  |  |
| 10 | h), q) | 156.500 | 156.500 | x | x |  |  |
| 70 | f), j) | 156.525 | 156.525 | Digital selective calling for distress, safety and calling |  |  |  |
| 11 |  | 156.550 | 156.550 |  | x |  |  |
| 71 |  | 156.575 | 156.575 |  | x |  |  |
| 12 |  | 156.600 | 156.600 |  | x |  |  |
| 72 | i) | 156.625 |  | x |  |  |  |
| 13 | k) | 156.650 | 156.650 | x | x |  |  |
| 73 | h), i) | 156.675 | 156.675 | x | x |  |  |
| 14 |  | 156.700 | 156.700 |  | x |  |  |
| 74 |  | 156.725 | 156.725 |  | x |  |  |
| 15 | g) | 156.750 | 156.750 | x | x |  |  |
| 75 | n) | 156.775 | 156.755 |  | x |  |  |

| Channel designator | Notes | Transmitting frequencies (MHz) |  | Inter-ship | Port operations and ship movement |  | Public corres-pondence |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ship stations | Coast stations |  | Single frequency | Two frequency |  |
| 16 | f) | 156.800 | 156.800 | DISTRESS, SAFETY AND CALLING |  |  |  |
| 76 | n) | 156.825 | 156.825 |  | x |  |  |
| 17 | g) | 156.850 | 156.850 | x | x |  |  |
| 77 |  | 156.875 |  | x |  |  |  |
| 18 | m) | 156.900 | 161.500 |  | x | x | x |
| 78 | m) | 156.925 | 161.525 |  |  | x | x |
| 19 | m) | 156.950 | 161.550 |  |  | x | x |
| 79 | m) | 156.975 | 161.575 |  |  | x | x |
| 20 | m) | 157.000 | 161.600 |  |  | x | x |
| 80 | m) | 157.025 | 161.625 |  |  | x | x |
| 21 | m) | 157.050 | 161.650 |  |  | x | x |
| 81 | m) | 157.075 | 161.675 |  |  | x | x |
| 22 | m) | 157.100 | 161.700 |  | x | x | x |
| 82 | m), o) | 157.125 | 161.725 |  | x | x | x |
| 23 | m), o) | 157.150 | 161.750 |  | x | x | x |
| 83 | m), o) | 157.175 | 161.775 |  | x | x | x |
| 24 | m), o) | 157.200 | 161.800 |  | x | x | x |
| 84 | m), o) | 157.225 | 161.825 |  | x | x | x |
| 25 | m), o) | 157.250 | 161.850 |  | x | x | x |
| 85 | m), o) | 157.275 | 161.875 |  | x | x | x |
| 26 | m), o) | 157.300 | 161.900 |  | x | x | x |
| 86 | m), o) | 157.325 | 161.925 |  | x | x | x |
| 27 |  | 157.350 | 161.950 |  |  | x | x |
| 87 |  | 157.375 | 157.375 |  | x |  |  |
| 28 |  | 157.400 | 162.000 |  |  | x | x |
| 88 |  | 157.425 | 157.425 |  | x |  |  |
| AIS 1 | f), l), p) | 161.975 | 161.975 |  |  |  |  |
| AIS 2 | f), l), p) | 162.025 | 162.025 |  |  |  |  |

Notes referring to the Table

General notes

MOD	EUR/10A14/188

e)	Administrations may apply 12.5 kHz channel interleaving on a non-interference basis to 25 kHz channels, in accordance with the most recent version of Recommendation ITU-R M.1084, provided:

–	it shall not affect the 25 kHz channels of the present Appendix maritime mobile distress and safety frequencies, especially the channels 06, 13, 15, 16, 17, and 70, nor the technical characteristics set forth in Recommendation ITU-R M.489-2 for those channels;

–	implementation of 12.5 kHz channel interleaving and consequential national requirements shall be subject to prior agreement between the implementing administrations and administrations whose ship stations or services may be affected.

Specific notes

MOD	EUR/10A14/189

f)	The frequencies 156.300 MHz (channel 06), 156.525 MHz (channel 70), 156.800 MHz (channel 16), 161.975 MHz (AIS1) and 162.025 MHz (AIS2) may also be used by aircraft stations for the purpose of search and rescue operations and other safety-related communication.-

MOD	EUR/10A14/190

l)	These channels (AIS 1 and AIS 2) are used for an automatic identification system capable of providing worldwide operation in accordance with ITU-R Recommendations, unless other frequencies are designated on a regional basis for this purpose.

MOD	EUR/10A14/191

m)	These channels may be operated as single frequency channels, subject to coordination with affected administrations.     (WRC-2000)

MOD	EUR/10A14/192

o)	These channels may be used to provide bands for new technologies, subject to coordination with affected administrations. Stations using these channels or bands for new technologies shall not cause harmful interference to, and shall not claim protection from, other stations operating in accordance with Article 5. (WRC-2000)

ADD	EUR/10A14/193

p)	Additionally, AIS 1 and AIS 2 may be used for satellite reception of AIS transmissions from stations in the maritime mobile service.

ADD	EUR/10A14/194

q)	When using these channels (10 and 11), all precautions should be taken to avoid harmful interference to channel 70.

SUP	EUR/10A14/195

APPENDIX  19

Technical characteristics of emergency position-indicating radiobeacons
operating on the carrier frequency 2 182 kHz

(See Appendix 13, Part A5)

MOD	EUR/10A14/196

RESOLUTION  18  (WRC-07)

Relating to the procedure for identifying and announcing the position of
ships and aircraft of States not parties to an armed conflict1

The World Radiocommunication Conference (Geneva, 2007),

considering

a)	that ships and aircraft encounter considerable risk in the vicinity of an area of armed conflict;

b)	that for the safety of life and property it is desirable for ships and aircraft of States not parties to an armed conflict to be able to identify themselves and announce their position in such circumstances;

c)	that radiocommunication offers such ships and aircraft a rapid means of self-identification and providing location information prior to their entering areas of armed conflict and during their passage through the areas;

d)	that it is considered desirable to provide a supplementary signal and procedure for use, in accordance with customary practice, in the area of armed conflict by ships and aircraft of States representing themselves as not parties to an armed conflict;

resolves

1	that the frequencies for urgency signal and messages specified in the Radio Regulations may be used by ships and aircraft of States not parties to an armed conflict for self-identification and establishing communications. The transmission will consist of the urgency or safety signals, as appropriate, described in Article 33 followed by the addition of the single group “NNN” in radiotelegraphy and by the addition of the single word “NEUTRAL” pronounced as in French “neutral” in radiotelephony. As soon as practicable, communications shall be transferred to an appropriate working frequency;

2	that the use of the signal as described in the preceding paragraph indicates that the message which follows concerns a ship or aircraft of a State not party to an armed conflict. The message shall convey at least the following data:

a)	call sign or other recognized means of identification of such ship or aircraft;

b)	position of such ship or aircraft;

c)	number and type of such ships or aircraft;

d)	intended route;

e)	estimated time en route and of departure and arrival, as appropriate;

f)	any other information, such as flight altitude, radio frequencies guarded, languages and secondary surveillance radar modes and codes;

3	that the provisions of Article 33 relating to urgency and safety transmissions, and medical transports shall apply as appropriate to the use of the urgency and safety signals, respectively, by such ship or aircraft;

4	that the identification and location of ships of a State not party to an armed conflict may be effected by means of appropriate standard maritime radar transponders. The identification and location of aircraft of a State not party to an armed conflict may be effected by the use of the secondary surveillance radar (SSR) system in accordance with procedures to be recommended by the International Civil Aviation Organization (ICAO);

5	that the use of the signals described above would not confer or imply recognition of any rights or duties of a State not party to an armed conflict or a party to the conflict, except as may be recognized by common agreement between the parties to the conflict and a non-party;

6	to encourage parties to a conflict to enter into such agreements,

requests the Secretary-General

to communicate the contents of this Resolution to the International Maritime Organization (IMO) and ICAO for such action as they may consider appropriate,

requests ITU-R

to recommend an appropriate signal in the digital selective calling system for use in the maritime mobile service and other appropriate information as necessary.

MOD	EUR/10A14/197

RESOLUTION  331  (Rev.WRC-07)

Transition to the Global Maritime Distress and Safety System (GMDSS)

The World Radiocommunication Conference (Geneva, 2007),

noting

that all ships subject to the International Convention for the Safety of Life at Sea (SOLAS), 1974, as amended, are required to be fitted for the Global Maritime Distress and Safety System (GMDSS),

noting further

a)	that a number of administrations have taken steps to implement the GMDSS also for classes of vessels not subject to SOLAS, 1974, as amended;

b)	that an increasing number of vessels not subject to SOLAS, 1974, as amended, are making use of the techniques and frequencies of the GMDSS prescribed in Chapter VII;

c)	that this Conference has amended Chapter VII to provide for maintaining interoperability between ships fitted for the GMDSS and ships not yet fully equipped for GMDSS;

d)	that there may be a need to maintain existing shore-based distress and safety services for reception of distress, urgency and safety calling by voice on VHF channel 16 for some years after this Conference so that vessels not subject to SOLAS, 1974, as amended and not yet using the techniques and frequencies of the GMDSS will be able to attract attention and obtain assistance from these services until such time as they are able to participate in the GMDSS;

e)	that the International Maritime Organization (IMO) is of the view that listening watch on VHF channel 16 by SOLAS ships, while at sea, should be required and kept for a foreseeable future with a view to providing:

–	a distress alerting and communication channel for non-SOLAS ships; and

–	bridge-to-bridge communications;

f)	that IMO has urged administrations to require all seagoing vessels under national legislation, and encourage all vessels voluntarily carrying VHF radio equipment to be fitted with facilities for transmitting and receiving distress alerts by digital selective calling (DSC) on VHF channel 70;

g)	that the Radio Regulations require GMDSS ships to keep watch on the appropriate digital selective calling (DSC) distress frequencies;

h)	that separate provisions in the existing Radio Regulations designate VHF channel 16 as the international channel for general calling by radiotelephony;

i)	that several administrations have established Vessel Traffic Service (VTS) systems and require their vessels to keep watch on local VTS channels;

j)	that ships that are required by SOLAS to carry a radio station have been equipped with DSC, and many vessels subject to national carriage requirements are also being equipped with DSC, but the majority of vessels that carry a radio station on a voluntary basis might not yet have DSC equipment;

k)	that similarly, many administrations have established distress and safety service based on DSC watchkeeping, but the majority of port stations, pilot stations and other operational coast stations might not yet have been equipped with DSC facilities;

l)	that Nos. 52.190 to 52.192 and 52.232 to 52.234 allow 2 182 kHz and channel 16 to be used for call and reply,

recognizing

a)	that, as indicated in noting further a), b), f), j) and k) above, stations in the maritime mobile service are increasingly making use of the frequencies and techniques of the GMDSS;

b)	that this Conference has adopted provisions for distress, urgency and safety calling by radiotelephony on VHF channel 16 and requiring ships, where practicable, to maintain watch on VHF channel 16;

c)	the need to maintain existing shore-based distress and safety services for reception of distress, urgency and safety calling by voice on VHF channel 16 for some years after this Conference so that vessels not subject to SOLAS, 1974, as amended and not yet using the techniques and frequencies of the GMDSS will be able to attract attention and obtain assistance from these services until such time as they are able to participate in the GMDSS;

d)	the need indicated in noting further d) above for maintaining existing shore-based distress, urgency and safety services on VHF channel 16,

resolves

1	to retain the provisions permitting use of VHF channel 16 and the frequency 2 182 kHz for general voice-calling;

2	to urge all administrations to assist in enhancing safety at sea by:

–	encouraging all vessels to finalize the transition to the GMDSS as soon as possible;

–	encouraging, where appropriate, establishment of suitable shore-based facilities for GMDSS, either on an individual basis or in cooperation with other relevant parties in the area;

–	encouraging all vessels carrying maritime VHF equipment to be fitted with DSC on VHF channel 70 as soon as possible, taking into account the relevant decisions of IMO;

–	encouraging vessels to limit their use of VHF channel 16 and the frequency 2 182 kHz for calling to the minimum necessary, noting the provisions of No. 52.239;

3	that coast stations forming part of shore-based arrangements in the area concerned for reception of distress calling by radiotelephony on VHF channel 16 should maintain an efficient watch on VHF channel 16. Such watch should be indicated in the List of Coast Stations;

4	that administrations may release their ship stations and coast stations from the listening watch on VHF channel 16 in respect of distress, urgency and safety calling by voice, in accordance with relevant decisions of IMO and ITU on aural watchkeeping requirements on channel 16, taking into account of the GMDSS radio systems available in the area concerned;

when doing so, administrations should:

–	inform IMO of their decisions and submit to IMO details on the area concerned;

–	inform the Secretary-General on the necessary details for inclusion in the List of Coast Stations,

instructs ITU-R

to monitor the development and changes to the GMDSS, in particular:

–	watchkeeping requirements;

–	distress alerting;

–	carriage requirements;

and report to a future World Radiocommunication Conference on when further rationalization of Chapter VII should be considered,

resolves further

that the Secretary-General should ensure that such arrangements and details regarding the area concerned be indicated in relevant maritime publications,

instructs the Secretary-General

to bring this Resolution to the attention of IMO, the International Civil Aviation Organization (ICAO) and the International Association of Marine Aids to Navigation and Lighthouse Authorities.

Reasons:	Several ships are not yet fully equipped for GMDSS. Revision of Resolution 331 is required to urge administrations to have their ships fitted with DSC and to manage the aural watchkeeping on channel 16. This European Common Proposal is dealing with this specific topic and is in line with the CPM Report.

______________

1	WRC-97 made editorial amendments to this Resolution.