| World Radiocommunication Conference (WRC-07) Geneva, 22 October - 16 November 2007 |  |
| --- | --- |
| PLENARY MEETING | Addendum 20 to Document 10-E |
|  | 22 May 2007 |
|  | Original: English |
| EUROPEAN COMMON PROPOSALS FOR THE WORK OF THE CONFERENCE |  |
| PART 20 |  |
| Agenda item 1.20 |  |

1.20	to consider the results of studies, and proposals for regulatory measures if appropriate regarding the protection of the EESS (passive) from unwanted emissions of active services in accordance with Resolution 738 (WRC-03)

CONTENTS

Page

Introduction		2

Sub-Part 20A – Protection of EESS (passive) from unwanted emissions within the 
band 1 400-1 427 MHz		3

Sub-Part 20B – Protection of EESS (passive) from unwanted emissions within the 
bands 23.6-24 GHz, 31.3-31.5 GHz, 50.2-50.4 GHz and 52.6-54.25 GHz		7

Introduction

ITU-R Task Group 1/7 conducted studies between the EESS (passive) and active services in certain adjacent or nearby bands, the results are contained in Recommendation ITU-R SM.1633. The results of this work were considered by WRC-03 under Agenda item 1.8.2.

CEPT made proposals to WRC-03 for the inclusion in the Radio Regulations of limits on the unwanted emissions of active services to protect the EESS (passive) in the bands 31.3-31.5 GHz and 52.6-54.25 GHz, and for continued studies on other specific bands with the results to be considered at WRC-07.

Many administrations supported regulatory measures for the protection of passive services, including EESS, at WRC-03; however, after difficult discussions up to the last few days of the Conference there was no agreement to take such action. The compromise solution was to continue the studies according to Resolution 738 and re-visit the issue at WRC-07.

Within WRC-07 Agenda item 1.20 covered by Resolution 738, a total of five EESS (passive) bands need to be addressed, i.e.:

1 400-1 427 MHz

23.6-24.0 GHz

31.3-31.5 GHz

50.2-50.4 GHz

52.6-54.25 GHz

The ECP on Agenda item 1.20 builds upon the results of work within TG 1/9 and has been split into two parts.

Sub-Part 20A

Protection of EESS (passive) from unwanted emissions within 
the band 1 400-1 427 MHz

Introduction

This sub-part deals with the issues under Agenda item 1.20 addressing the EESS (passive) band 1 400-1 427 MHz.

The results of the compatibility analyses are mentioned in Table 1. It should be noted that in most cases equipment complying with relevant widely accepted standard, when available, or category B spurious limits of Recommendation ITU-R SM.329 would also comply with the limits of the table below.

TABLE 1

Results of the band-by-band studies

| Passive band | Active band | Active service | Measure to protect passive band* |
| --- | --- | --- | --- |
| 1 400-1 427 MHz | 1 350-1 400 MHz | RL | Limit unwanted emissions of radiolocation stations into the passive band to –29 dB(W/27 MHz) |
| 1 400-1 427 MHz | 1 350-1 400 MHz | FS | Limit unwanted emissions of FS stations into the passive band to –45 dB(W/27 MHz) |
| 1 400-1 427 MHz | 1 350-1 400 MHz | MS | Limit unwanted emissions of MS stations into the passive band to –60 dB(W/27 MHz) except transportable radio relays1 limited to –45 dB(W/27 MHz) |
| 1 400-1 427 MHz | 1 427-1 429 MHz | SO (E to s) | Limit unwanted emissions of SO earth stations into the passive band to –36 dB(W/27 MHz) |
| 1 400-1 427 MHz | 1 427-1 452 MHz | MS | Limit unwanted emissions of MS stations into the passive band to –60 dB(W/27 MHz) except transportable radio relays1 limited to –45 dB(W/27 MHz) and aeronautical telemetry2 limited to –28 dB(W/27 MHz) |
| 1 400-1 427 MHz | 1 427-1 452 MHz | FS | Limit unwanted emissions of FS stations into the passive band to –45 dB(W/27 MHz) |
| * The unwanted emission power level should be understood as that level prior at the antenna port of the active service station (i.e. not including the antenna gain). 1 Some administrations operate transportable radio relay systems in the mobile service, while others operate such systems in the fixed service. 2 The band 1 429-1 435 MHz is also allocated to the aeronautical mobile service in eight Region 1 administrations on a primary basis exclusively for the purposes of aeronautical telemetry within their national territory (RR No. 5.342). |  |  |  |

Issue (A)	resolves 1 and 3 (Resolution 738)

Resolves 1 reads as follows: “to invite ITU-R to continue or to initiate studies on the compatibility analyses between EESS (passive) and the corresponding active services as listed in the Table with a view to updating Recommendation ITU-R SM.1633 or developing additional Recommendations”, whereas resolves 3 is on the consideration of regulatory measures for WRC-07 based on the previous resolves.

The proposal is based on a single entry unwanted emission mandatory power limit applicable to a reference bandwidth in the EESS (passive) band for each corresponding active service taking into account the results of the compatibility analyses: those operational limits are proposed to be included into a footnote in Article 5 similar to No. 5.482. Those limits are derived from part or all of Table 1 depending on the result of studies documented in the Report ITU-R SM.2092.

In most cases, these studies have included equipments whose compliance with relevant widely accepted standard, when available, or category B spurious limits of Recommendation ITU-R SM.329, when applicable ensure the compliance with the limits in most cases. They have also included mitigation assumptions when needed, therefore providing an equitable burden sharing between active and passive services.

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/10A20/1

1 300-1 525 MHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 1 350-1 400 FIXED ADD 5.A20 MOBILE ADD 5.B20 RADIOLOCATION ADD 5.C20 | 1 350-1 400 RADIOLOCATION ADD 5.C20 |  |
| 5.149 5.338 5.339 5.339A | 5.149 5.334 5.339 5.339A |  |
| 1 400-1 427 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 5.341 |  |  |
| 1 427-1 429 SPACE OPERATION (Earth-to-space) ADD 5.D20 FIXED ADD 5.A20 MOBILE except aeronautical mobile ADD 5.B20 5.341 |  |  |
| 1 429-1 452 FIXED ADD 5.A20 MOBILE except aeronautical mobile ADD 5.B20 | 1 429-1 452 FIXED ADD 5.A20 MOBILE 5.343 ADD 5.B20 |  |
| 5.339A 5.341 5.342 | 5.339A 5.341 |  |

ADD	EUR/10A20/2

5.A20	Unwanted emissions from stations of the fixed service brought into use after [date of entry into force of the revised Radio Regulations (WRC-07)] shall be limited to a maximum unwanted emission power supplied at the antenna port of –45 dBW within the passive band 1 400-1 427 MHz. The Bureau shall make no examination or finding with respect to this limit under Articles 9 and 11.

ADD	EUR/10A20/3

5.B20	Unwanted emissions from stations of the mobile service (except transportable radio relays and aeronautical telemetry) brought into use after [date of entry into force of the revised Radio Regulations (WRC-07)] shall be limited to a maximum unwanted emission power supplied at the antenna port of –60 dBW within the passive band 1 400-1 427 MHz. Unwanted emissions from transportable radio relays and aeronautical telemetry brought into use after [date of entry into force of the revised Radio Regulations (WRC-07)] shall be limited respectively to a maximum unwanted emission power supplied at the antenna port of –45 dBW and –28 dBW within the passive band 1 400-1 427 MHz. The Bureau shall make no examination or finding with respect to these limits under Articles 9 and 11.

ADD	EUR/10A20/4

5.C20 	The mean power of unwanted emissions of stations of the radiolocation service operating in the band 1 215-1 400 MHz and brought into use after 1 January 2015 shall be limited to a maximum of –29 dB(W/27 MHz) in the band 1 400-1 427 MHz. Mean power is to be understood here as the total power measured at the antenna port (or an equivalent thereof) in the band 1 400-1 427 MHz, averaged over a period in the order of 5 s. The Bureau shall make no examination or finding with respect to this limit under Articles 9 and 11.

ADD	EUR/10A20/5

5.D20	Unwanted emissions from stations of the space operations service brought into use after [date of entry into force of the revised Radio Regulations (WRC-07)] shall be limited to a maximum unwanted emission power supplied at the antenna port of –36 dBW within the passive band 1 400-1 427 MHz. The Bureau shall make no examination or finding with respect to this limit under Articles 9 and 11.

Reasons:	See Introduction.

Sub-Part 20B

Protection of EESS (passive) from unwanted emissions within the bands 
23.6-24 GHz, 31.3-31.5 GHz, 50.2-50.4 GHz and 52.6-54.25 GHz

Introduction

This sub-part deals with the issues under Agenda item 1.20 covering the bands from 23.6-24 GHz to 52.6-54.25 GHz.

TABLE 1

Results of the band by band studies

| Passive band | Active band | Active service | Measure to protect passive band* |
| --- | --- | --- | --- |
| 23.6-24.0 GHz | 22.55-23.55 GHz | ISS | Limit unwanted emissions of ISS space stations into the passive band to –46 dB(W/200 MHz) |
| 31.3-31.5 GHz | 30.0-31.0 GHz | FSS (E-to-s) | Limit unwanted emissions of FSS earth stations into the passive band to –20 dB(W/200 MHz) |
| 31.3-31.5 GHz | 31-31.3 GHz | FS | Limit unwanted emissions of FS stations into the passive band to –38 dB(W/100 MHz) according to Resolution 738 (WRC-03) |
| 50.2-50.4 GHz | 47.2-50.2 GHz Regions 2 and 3, 49.44-50.2 GHz Region 1 | FSS (E to s) | Limit unwanted emissions of the FSS earth stations into the passive band to –20 dB(W/200 MHz) |
| 50.2-50.4 GHz | 50.4-51.4 GHz | FSS (E to s) | Limit unwanted emissions of the FSS earth stations into the passive band to –20 dB(W/200 MHz) |
| 52.6-54.25 GHz | 51.4-52.6 GHz | FS | Limit unwanted emissions of FS stations into the passive band to –33 dB(W/100 MHz) (from Resolution 738) |
| * The unwanted emission power level should be understood as that level supplied to the antenna port of the active service station (i.e. not including the antenna gain). NOTE 1 – It should be noted that in most cases equipment complying with relevant widely accepted standard, when available, or category B spurious limits of Recommendation ITU-R SM.329 would also comply with the limits of the table above. |  |  |  |

Issue (A)	resolves 1 and 3 (Resolution 738)

Resolves 1 reads as follows: “to invite ITU-R to continue or to initiate studies on the compatibility analyses between EESS (passive) and the corresponding active services as listed in the Table with a view to updating Recommendation ITU-R SM.1633 or developing additional Recommendations”, whereas resolves 3 is on the consideration of regulatory measures for WRC-07 based on the previous resolves.

The proposal is based on a single entry unwanted emission mandatory power limit applicable to a reference bandwidth in the EESS (passive) band for each corresponding active service taking into account the results of the compatibility analysis: those operational limits are proposed to be included in the Radio Regulations through a resolution. A footnote in Article 5 similar to No. 5.347A would direct towards this resolution. It is to be noted that the regulatory conditions of

those operational limits (non-retroactivity and non-examination by BR) would also be included in this Resolution. Those limits are derived from part or all of Table 1 depending on the result of studies documented in the Report ITU-R SM.2092.

These studies have included equipments whose compliance with relevant widely accepted standard, when available, or category B spurious limits of Recommendation ITU-R SM.329 ensure the compliance with the limits in most cases. They have also included mitigation assumptions when needed, therefore providing an equitable burden sharing between active and passive services.

Issue (B)	resolves 2 and 3 (Resolution 738)

Resolves 2 reads as follows: “to invite ITU-R to further study the impact of implementing the values provided in considering f) and g) for unwanted emissions of fixed-service systems operating in Regions 2 and 3, taking into account that the impact on fixed-service systems in Region 1 has already been investigated”, whereas resolves 3 is on the consideration of regulatory measures for WRC-07 based on the previous resolves.

The proposed method for Issue (B) is in line with the part related to these bands for the European Common Proposal at WRC-03 for Agenda item 1.8.2 (WRC-03) with the exception of that the limits are now proposed to be included in a resolution.

ARTICLE  5

Frequency allocations

Section IV  –  Table of Frequency Allocations
(See No. 2.1)

MOD	EUR/10A20/6

22-24.75 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 22.55-23.55 FIXED INTER-SATELLITE ADD 5.E20 MOBILE 5.149 |  |  |
| 23.55-23.6 FIXED MOBILE |  |  |
| 23.6-24 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |

MOD	EUR/10A20/7

29.9-34.2 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 30-31 FIXED-SATELLITE (Earth-to-space) ADD 5.E20 MOBILE-SATELLITE (Earth-to-space) Standard frequency and time signal-satellite (space-to-Earth) 5.542 |  |  |
| 31-31.3 FIXED 5.543A ADD 5.E20 MOBILE Standard frequency and time signal-satellite (space-to-Earth) Space research 5.544 5.545 5.149 |  |  |
| 31.3-31.5 EARTH EXPLORATION-SATELLITE (passive) RADIO ASTRONOMY SPACE RESEARCH (passive) 5.340 |  |  |

MOD	EUR/10A20/8

40-47.5 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 47.2-47.5 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.E20 MOBILE 5.552A |  |  |

MOD	EUR/10A20/9

47.5-51.4 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 47.5-47.9 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.E20 (space-to-Earth) 5.516B 5.554A MOBILE | 47.5-47.9 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.E20 MOBILE |  |
| 47.9-48.2 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.E20 MOBILE 5.552A |  |  |
| 48.2-48.54 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.E20 (space-to-Earth) 5.516B 5.554A 5.555B MOBILE | 48.2-50.2 FIXED FIXED-SATELLITE (Earth-to-space) 5.516B 5.552 ADD 5.E20 MOBILE |  |
| 48.54-49.44 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.E20 MOBILE 5.149 5.340 5.555 |  |  |
| 49.44-50.2 FIXED FIXED-SATELLITE (Earth-to-space) 5.552 ADD 5.E20 (space-to-Earth) 5.516B 5.554A 5.555B MOBILE | 5.149 5.340 5.555 |  |
| 50.2-50.4 EARTH EXPLORATION-SATELLITE (passive) SPACE RESEARCH (passive) 5.340 |  |  |
| 50.4-51.4 FIXED FIXED-SATELLITE (Earth-to-space) ADD 5.E20 MOBILE Mobile-satellite (Earth-to-space) |  |  |

MOD	EUR/10A20/10

51.4-55.78 GHz

| Allocation to services |  |  |
| --- | --- | --- |
| Region 1 | Region 2 | Region 3 |
| 51.4-52.6 FIXED ADD 5.E20 MOBILE 5.547 5.556 |  |  |

ADD	EUR/10A20/11

5.E20	In the bands 22.55-23.55 GHz, 30-31 GHz, 31-31.3 GHz, 50.4-51.4 GHz, 47.2-50.2 GHz (Regions 2 and 3), 49.44-50.2 GHz (Region 1) and 51.4-52.6 GHz Resolution [EUR/10A20/12] (WRC-07) applies.

Reasons:	See Introduction.

NOTE – In addition, this Resolution could also include all the band pairs for the 1.4 GHz issue where mandatory limits exists applying the same regulatory and procedural considerations.

ADD	EUR/10A20/12

DRAFT RESOLUTION  [EUR/10A20/12]  (WRC-07)

Compatibility analyses between the Earth exploration-satellite 
service (passive) and active services

The World Radiocommunication Conference (Geneva, 2007),

considering

a)	that primary allocations have been made to various space services in the Earth-to-space direction such as the fixed-satellite service (FSS), inter-satellite service (ISS)  and/or to terrestrial services such as the fixed service (FS) hereafter referred to as “active services”, in bands adjacent or nearby to bands allocated to the Earth exploration-satellite service (EESS) (passive);

b)	that unwanted emissions from active services may cause unacceptable interference to the EESS (passive);

c)	that for technical or operational reasons, the general limits in Appendix 3 may be insufficient in protecting the EESS (passive) in specific bands;

d)	that, in many cases, the frequencies used by the EESS (passive) sensors are chosen to study natural phenomena producing radio emissions at frequencies fixed by the laws of nature, and therefore shifting frequency to avoid or mitigate interference problems may not be possible;

e)	that it is necessary to ensure an equitable burden sharing for achieving compatibility between active and passive services operating in adjacent or nearby bands,

noting

a)	that frequency bands covered by RR No. 5.340 which prohibits all emissions need to be fully protected as they are unique natural resources;

b) 	that Report ITU-R SM.2092  provides a methodology for conducting the compatibility studies between active and passive services operating in adjacent and nearby bands and develops all relevant scenarios;

c)	that Recommendation ITU-R RS.1029-2 provides the interference criteria for satellite passive remote sensing,

resolves

1	that unwanted emissions from stations, brought into use after the [date of entry into force of the revised Radio Regulations (WRC-07)], in the bands and services listed in Table 1-1 below, shall not exceed the corresponding limits in the table;

2	that the Bureau shall make no examination or finding with respect to this Resolution under either Article 9 or 11.

TABLE 1-1

| Passive band | Active band | Active service | Measure to protect passive band* |
| --- | --- | --- | --- |
| 23.6-24.0 GHz | 22.55-23.55 GHz | ISS | Limit unwanted emissions of ISS space station into the passive band to –46 dBW in 200 MHz |
| 31.3-31.5 GHz | 30.0-31.0 GHz | FSS (E to s) | Limit unwanted emissions of FSS earth stations into the passive band to –20 dB(W/200 MHz) |
| 31.3-31.5 GHz | 31-31.3 GHz | FS | Limit unwanted emissions of FS stations into the passive band to –38 dB(W/100 MHz) |
| 50.2-50.4 GHz | 47.2-50.2 GHz (Regions 2 and 3), 49.44-50.2 GHz (Region 1) | FSS (E to s) | Limit unwanted emissions of the FSS earth stations into the passive band to –20 dB(W/200 MHz ) |
| 50.2-50.4 GHz | 50.4-51.4 GHz | FSS (E to s) | Limit unwanted emissions of the FSS earth stations into the passive band to –20 dB(W/200 MHz) |
| 52.6-54.25 GHz | 51.4-52.6 GHz | FS | Limit unwanted emissions of FS stations into the passive band to –33 dB(W/100 MHz) |
| * The unwanted emission power level is defined as the level at the antenna port of the active service station (i.e. not including the antenna gain). |  |  |  |

SUP	EUR/10A20/13

RESOLUTION  738  (WRC-03)

Compatibility analyses between the Earth exploration-satellite 
service (passive) and active services

Reasons:	Studies are completed.

_____________