| XII MEETING OF PERMANENT CONSULTATIVE COMMITTEE II: RADIOCOMMUNICATIONS INCLUDING BROADCASTING June 2-5, 2009 Ottawa, Canada |  | OEA/Ser.L/XVIII.4.2 CCP.II-RADIO/doc. 1853/09 15 May 2009 Original: English |  |
| --- | --- | --- | --- |
|  | ### DRAFT PRELIMINARY VIEW 2011 WORLD RADIOCOMMUNICATION CONFERENCES |  |  |
|  | (Document submitted by the delegation of United States of America) |  |  |

PRELIMINARY VIEWS ON WRC-11

Agenda Item 1.6 (Res. 950)

AGENDA ITEM 1.6:  to review No. 5.565 of the Radio Regulations in order to update the spectrum use by the passive services between 275 GHz and 3 000 GHz, in accordance with Resolution 950 (Rev.WRC-07), and to consider possible procedures for free-space optical-links, taking into account the results of ITU-R studies, in accordance with Resolution 955 (WRC-07)

ISSUE:  The purpose of Resolution 950 (Rev. WRC-07) is to review No. 5.565, excluding frequency allocations, in order to update spectrum use between 275 and 3 000 GHz by the passive services.  Currently, No. 5.565 describes the need for passive observations of spectral line emissions and spectral windows in various bands throughout the 275 – 1 000 GHz range by the radio astronomy service (RAS), the Earth exploration-satellite service (passive) (EESS), and the space research service (passive) (SRS).  The footnote also describes the potential for additional spectral line and continuum bands in this range to be identified in the future.  Resolution 950 (Rev. WRC-07) extends its range of consideration to 275 – 3 000 GHz for RAS, EESS (passive), and SRS (passive) use, and invites ITU-R to conduct studies toward modifying No. 5.565.

BACKGROUND:  The current Table of Frequency Allocations establishes allocations at frequencies between 9 kHz and 275 GHz.  No allocations presently exist above 275 GHz, although an entry in the Table for the range 275 – 1 000 GHz contains a reference to No. 5.565:

5.565	The frequency band 275-1 000 GHz may be used by administrations for experimentation with, and development of, various active and passive services. In this band a need has been identified for the following spectral line measurements for passive services:

– radio astronomy service: 275-323 GHz, 327-371 GHz, 388-424 GHz, 426-442 GHz, 453-510 GHz, 623-711 GHz, 795-909 GHz and 926-945 GHz;

– Earth exploration-satellite service (passive) and space research service (passive): 275-277 GHz, 294-306 GHz, 316-334 GHz, 342-349 GHz, 363-365 GHz, 371-389 GHz, 416-434 GHz, 442-444 GHz, 496-506 GHz, 546-568 GHz, 624-629 GHz, 634-654 GHz, 659-661 GHz, 684-692 GHz, 730-732 GHz, 851-853 GHz and 951-956 GHz.

Future research in this largely unexplored spectral region may yield additional spectral lines and continuum bands of interest to the passive services. Administrations are urged to take all practicable steps to protect these passive services from harmful interference until the date when the allocation Table is established in the above-mentioned frequency band.    (WRC-2000)

Passive services currently utilize portions of the 275 – 3 000 GHz range for scientific observation of both spectral line and continuum emissions.  Resolution 950 (Rev. WRC-07) resolves to review No. 5.565 to update the spectrum use between 275 and 3 000 GHz by the passive services, but specifically excludes allocations in this range.  Resolution 950 (Rev. WRC-07), considers that the present use of the bands between 275 and 3 000 GHz is mainly related to the passive services, however, with anticipated technology development, the bands may become increasingly important for suitable active service applications.

U.S. VIEW:  The United States supports the modification of No. 5.565 to include all appropriate bands of interest to RAS, EESS (passive), and SRS (passive) in the range 275 – 3 000 GHz based upon studies being conducted in Study Group 7.  The identification of bands for use by the passive services above 275 GHz should not preclude the use of these bands by the active services in the future.

__________________

This preliminary view only addresses the first part of the agenda item (passive services between 275 – 3 000 GHz), hereafter referred to as Agenda Item 1.6 (Res 950).  The second part of the agenda item (free-space optical links), referred to as Agenda Item 1.6 (Res 955), is addressed in a separate document.